WebAPI Development Doc.

Version 1.0(Build 2019-12-18)

All rights reserved

Forward

Thank you very much for using our equipment, we will provide you with the best service.

This document may contain technically inaccurate areas or text, typographical errors, welcome corrections. We will update the contents of the document regularly.

Revision record

| Date | Content Revision | Modified person |
| --- | --- | --- |
| 2019.12.18 | New |  |
| 2025.08.04 | 1、Modified 2.5.22 web local upgrade; 2. Added 2.3.5 latitude and longitude information |  |
| 2025.08.06 | 1、Added 2.7.12 one-click disarm configuration; 2. Added 2.3.15 basic device information; Updated 2.9.3 with WDR Level and LampSourceMode; Added 2.6.5 IPV6; Added 2.9.2 and 2.9.4 3D and 2D noise reduction, stripe suppression, exposure compensation, and metering mode. | s |
| 2025.08.07 | 1、Update 2.6.5 add NetDHCP; 2、add 2.9.7 White balance interface |  |
| 2025.08.08 | 1、Update 2.7.10 add bDrawBox；2、update 2.3.7add iDateFormat and iDateSeparator；3、add 2.3.16 rtsp http auth；4、add 2.3.17和2.3.18 One-click restart,Reset |  |
| 2025.08.11 | 1、add 2.5.25 |  |
| 2025.08.12 | 1、update 2.5.12 add Multicast；2、update 2.10.2；3、update 2.5.23 add streamType and protocol |  |
| 2025.08.14 | 1、6.1 Add configuration import/export cmdWEBSOCKET_CMD_EXPORT_SYS_CONFIG_PACK = 259, // Package export configuration WEBSOCKET_CMD_IMPORT_SYS_CONFIG = 261,//Import configuration; 2. Update the 2.2.7 Table. Resolution Type list, change parameters for QCIF/960H/5M, and add H5M, 4K, 8K, 2560x1536, 2560x1920, 2880x1620; |  |
| 2025.08.18 | 4.2 statusString Add identification codes for corresponding WEB translations to facilitate WEB translation into various languages; any modifications need to be synchronized with the WEB. |  |
| 2025.09.01 | 2.3.14 Modify audio input and output fields |  |
| 2025.10.10 | 2.5.27 /CGI/NetPlatform/net4g Added a new APN configuration interface. Whether this page is displayed is determined by the field SupportMobileNetwork in 2.2.1/CGI/Ability/abilitysystemfunction. |  |
| 2025.10.13 | 2.5.27 /CGI/NetPlatform/net4g Add ICCID display; this field is read-only and cannot be edited. |  |
| 2025.10.20 | 2.9.1 /CGI/Preview/prevcover/channels/ Added a setting for the 4G OSD switch (SignalTitleAttribute). Whether this 4G OSD switch is displayed is determined by the Support Signal Title field in 2.2.1/CGI/Ability/ability system function. |  |
| 2025.10.21 | /CGI/Record/schedulelist Get which days have recordings and display them on the recording calendar. |  |

CONTENTS

1.	Introduction	7

1.1. Overview	7

1.2. JSON	7

2.	Service order	8

2.1. Login	8

2.1.1.	/CGI/Security/sessionLogin	8

2.1.2.	/CGI/Security/SelfExt/userCheck	9

2.1.3.	/CGI/Security/sessionHeartbeat	10

2.1.4.	/CGI/Security/DeviceInitCheck	10

2.1.5.	/CGI/Security/DeviceInitOption	10

2.1.6.	/CGI/Security/sessionForget	11

2.1.7.	/CGI/Security/SelfExt/forgetCheck	11

2.2. Ability	12

2.2.1.	/CGI/Ability/abilitysystemfunction	13

2.2.2.	/CGI/Ability/abilitycomm	16

2.2.3.	/CGI/Ability/abilityptz	17

2.2.4.	/CGI/Ability/abilityuart	17

2.2.5.	/CGI/Ability/abilitycamera	18

2.2.6.	/CGI/Ability/devdesc	18

2.2.7.	/CGI/Ability/abilityencode	19

2.2.8.	/CGI/Ability/abilityencodefps	24

2.3. General Configure	25

2.3.1.	/CGI/General/initgeneral	25

2.3.2.	/CGI/General/supportlanguage	26

2.3.3.	/CGI/General/general	26

2.3.4.	/CGI/General/location	27

2.3.5.	/CGI/General/ntp	28

2.3.6.	/CGI/General/time	30

2.3.7.	/CGI/General/autoMaintainConfig	31

2.3.8.	/CGI/Custom/comm	31

2.3.9.	/CGI/PTZ/rs485/channels/<channelNo>	32

2.3.10.	/CGI/PTZ/ptzcfg/channels/0	33

2.3.11.	/CGI/General/guiset	34

2.3.12.	/CGI/General/videoout	35

2.3.13.	/CGI/General/audioinout	35

2.4. Encode	36

2.4.1.	/CGI/Encode/encode/channels/<channelNo>	36

2.5. Net Service API Functions	39

2.5.1.	/CGI/NetService/ipfilter	39

2.5.2.	/CGI/NetService/ddns	40

2.5.3.	/CGI/NetService/ddnsstatus	41

2.5.4.	/CGI/NetService/email	42

2.5.5.	/CGI/NetService/emailtest	44

2.5.6.	/CGI/NetService/upnp	44

2.5.7.	/CGI/NetPlatform/cloud	45

2.5.8.	/CGI/NetService/ftp	45

2.5.9.	/CGI/NetService/ftpPath	46

2.5.10.	/CGI/NetService/ftptest	47

2.5.11.	/CGI/NetService/pppoe	47

2.5.12.	/CGI/NetService/rtsp	48

2.5.13.	/CGI/NetService/https	49

2.5.14.	/CGI/NetService/certmanager/exportrootcert	50

2.5.15.	/CGI/NetService/certmanager/svrcertinfo	51

2.5.16.	/CGI/NetService/certmanager/removesvrcert	52

2.5.17.	/CGI/NetService/certmanager/importsvrcert	52

2.5.18.	/CGI/NetService/certmanager/exporsvrtcert	54

2.5.19.	/CGI/NetService/otaupgrade	55

2.5.20.	/CGI/NetService/upgradestate	55

2.5.21.	/CGI/NetService/httpupgrade	56

2.5.22.	/CGI/Upgrade/init	57

2.5.23.	/CGI/Upgrade/Download	57

2.5.24.	/CGI/Upgrade/finalize	58

2.5.25.	/CGI/Upgrade/cancel	59

2.5.26.	/CGI/NetService/librtmp	59

2.6. Network	60

2.6.1.	/CGI/Network/netcommon	60

2.6.2.	/CGI/Network/dhcp	61

2.6.3.	/CGI/Network/mtu	62

2.6.4.	/CGI/Network/dns	62

2.7. Alarm	62

2.7.1.	/CGI/Alarm/motion/channels/<ID>	63

2.7.2.	/CGI/Alarm/blind/channels/<ID>	63

2.7.3.	/CGI/Alarm/alarmin/channels/<ID>	64

2.7.4.	/CGI/Alarm/alarmout	64

2.7.5.	/CGI/Alarm/abnormal/nodisk	64

2.7.6.	/CGI/Alarm/abnormal/diskerr	65

2.7.7.	/CGI/Alarm/abnormal/disknospace	65

2.7.8.	/CGI/Alarm/abnormal/netdisconn	66

2.7.9.	/CGI/Alarm/abnormal/ipconflict	66

2.7.10.	/CGI/Alarm/region/channels/<ID>	66

2.7.11.	/CGI/Alarm/crossline/channels/<ID>	67

2.8. UserManager	67

2.8.1.	/CGI/UserManager/grouplist	68

2.8.2.	/CGI/UserManager/userlist	68

2.8.3.	/CGI/UserManager/users/“username”	69

2.8.4.	/CGI/UserManager/users/”encrypted(“username”&“password”)”	70

2.8.5.	/CGI/UserManager/users/“username”	71

2.8.6.	/CGI/UserManager/groups/”groupname”	71

2.8.7.	/CGI/UserManager/groups/”groupname”	71

2.8.8.	/CGI/UserManager/groups/”groupname”	72

2.9. CameraParam	72

2.9.1.	/CGI/Preview/prevcover/channels/<ch.no>	72

2.9.2.	/CGI/Preview/colorset/channels/<ch.no>	73

2.9.3.	/CGI/Custom/cameraparam	74

2.9.4.	/CGI/Custom/cameraispparam	75

2.9.5.	/CGI/Custom/cameraprofile	76

2.9.6.	/CGI/Preview/texttitle/channels/<ch.no>	77

2.10. Video Manager	77

2.10.1.	/CGI/Record/record/channels/<ch.no>	77

2.10.2.	/CGI/Record/snapshot/channels/<ch.no>	79

2.10.3.	/CGI/Record/storage+<info/opt>	80

2.11. PTZ fuction	81

2.11.1.	/CGI/Ability/abilitycamera	81

2.11.2.	/CGI/PTZ/AFAbility	82

2.11.3.	/CGI/PTZ/AFControl	82

2.11.4.	/CGI/PTZ/PTZControl	84

3.	Normal JSON structure	86

3.1.	TimeSection	86

3.2.	Server	87

3.3.	EventHandler	88

3.4.	AuthorityList	89

4.	Error Code	91

4.1.	StatusCode	91

4.2.	Statusstring	92

5.	Function through OCX	92

6.	Web Socket Server	93

6.1.	General Cmd	93

6.2.	General Resp	94

6.3.	Video Play	94

6.4.	Record Play	94

6.5.	Record Play Ctrl	95

6.6.	Record Dowload	95

6.7.	Remote Upgrade	95

6.8.	Camera Encode	95

6.9.	Get Camera Overlay	95

6.10.	Set Camera Overlay	95

6.11.	Get Camera Privacy Mask	95

6.12.	Set Camera Privacy Mask	96

6.13.	Get Camera Motion Detect	96

6.14.	Set Camera Motion Detect	96

6.15.	Set Camera PTZ Ctrl	96

7.	Customize function	96

8.	Device’s password init	96

When the device is initialized to the factory state, before you login on web, you need to modify password at first.	96

- Introduction

  - Overview

Welcome to use our web API development documents. This document specifies an interface that allows our equipment to communicate with various IP media devices in a standardized manner.

Our company customizes private CGI service commands according to the CGI service model and various IP media devices communicate via the HTTP protocol. The protocol data is currently encoded in the JSON format.

  - JSON

JSON：JavaScript Object Notation

JSON is used to store and transfer, similar with XML。

JSON is smaller and quicker the XML。

{ "employees": [ { "firstName":"Bill" , "lastName":"Gates" }, { "firstName":"George" , "lastName":"Bush" }, { "firstName":"Thomas" , "lastName":"Carter" } ] }

示例：

JSON Writing format：key/value。key/value should be like this："firstName":"Bill"

JSON ：

- number（int or float）

- string（“” contain）

- Logic value（true or false）

- Array（Square brackets）

- object（curly braces）

- null

For the example,object "employees" include three array. Every mean one people.

Detailed JSON instructions, etc., can be viewed online

- Service order

The following table describes the service commands of our private CGI service model. The Json string is too long ,so we just put out the structure for you to understand the mean of parameter.

| URL | Description |
| --- | --- |
| /CGI | Root directory |
| /CGI/Security/ | Login, session check |
| /CGI/Ability | Equipment capacity set |
| /CGI/General | General configuration |
| /CGI/Encode | Coding configuration |
| /CGI/Record | Video configuration |
| /CGI/NetSerice | Network service configuration, including DDNS, E-mail, etc. |
| /CGI/Network | Universal network configuration |
| /CGI/Alarm | Alarm configuration |
| /CGI/PTZ | PTZ configuration |
| /CGI/UserManager | User configuration |
| /CGI/Preview | Preview configuration |
| /CGI/Camera | Camera configuration |

The device resource matched by the implemented service command.

  - Login

The Login process:

First: Use the sessionLogin api to get the “salt”,”sessionID”

Second: userCheck api will use the “salt”, password, the two string together use MD5 to get the changed password string, and send to the device. Device will use the sessionID to get the “salt” in the device, also use MD5 to get the password. Check the two to make sure if the password is right or error.

| Order | description | GET | POST |
| --- | --- | --- | --- |
| sessionLogin | The first step for login | √ |  |
| SelfExt/userCheck | The second step for login,check the user |  | √ |
| sessionHeartbeat | Heartbeat to make sure device is alive |  | √ |

    - /CGI/Security/sessionLogin

Sample:

URL: http://192.168.1.41/CGI/Security/sessionLogin?timestamp:1542282790916

Cookie: updateTips=true; language=en (show the language is English )

The reply text is the follow Json in the table：

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "salt": "NU1Om542UDi14Fx1UoElfSJl4tYqk1A2", | For the double check of password use for later |  |
| "sessionID": "p9W4w4h846kmc2o1", | sessionID,use for the next command of usercheck |  |
| "statusString": "OK", | See the return code |  |
| "statusValue": 200 |  |  |
| } |  |  |

    - /CGI/Security/SelfExt/userCheck

Sample:

URL: http://192.168.1.41/CGI/Security/SelfExt/userCheck

The Send body is:

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Username": "admin", | The User name |  |
| "Password": "uYLPg5A7", | MD5(salt + MD5(password)) |  |
| "Sessionid": "p9W4w4h846kmc2o1" | The reply sessionID“sessionLogin” API |  |
| } |  |  |

The reply body is:

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "language": 100, | No care now, before we design the device make sure the language,but now the web make sure the language. | d |
| "sessionID": "t70nilyO9LW87hoy", | The real sessionID, need to use all the later API command |  |
| "statusString": "OK", | See the return code |  |
| "statusValue": 200, |  |  |
| "leftLockTime":1800, | User lock duration. How long does it take to log in again. | int |
| "leftTryTimes":5, | When the password is wrong, you can log in times. | int |
| "tcpport": 34567 | The tcp port for ocx to login |  |
| } |  |  |

    - /CGI/Security/sessionHeartbeat

The API should always sent every 30 second during the lifetime.And if 2 Minutes send failed, Web will determined to be disconnected.

See the Sample, the webSessionID in the Cookie is the reply value from the API “userCheck”.

Sample:

URL: http://192.168.1.41/CGI/Security/sessionHeartbeat?timestamp:1542282824831

Cookie: updateTips=true; language=en; webSessionID=t70nilyO9LW87hoy; hostport=34567

    - /CGI/Security/DeviceInitCheck

When the device is initialized to the factory state, before you login on web, you need to modify password at first.

Used to check up whether you need to init befor login.

Sample:

URL: http://192.168.1.41/ CGI/Security/DeviceInitCheck

Type: GET(only)

| Json | Description | Value |
| --- | --- | --- |
| “Init”:1 | 0-need to init; 1-no need to init | Int 0/1 |
| "OEMID":0 | Not in use now |  |
| "ProductModel": "HI-DL31", | product model | String |
| "statusString" : "OK" | Status description | String |
| statusValue:200 | Status description, 200=check over, others=check failed | Int |

    - /CGI/Security/DeviceInitOption

Encrypted the username and new password, the PUT to device.

Sample:

URL:  http://192.168.1.41/ CGI/Security/ DeviceInitOption

Type: POST(only)

The API is used to initialize the device password. This can only do if the device is not initialized.

This is an very important , So the send content is encrypted. The whole process , do not say here.

Another document only giveed with our consent.

Send Content：

{

"Content":"3LhaJa/tve+30YZgfFX1DzmJ6edcLyasVIldHvUFsg0A1Wn0EjMlvkzL/v+AD1K0Un+9lxKDmevBI+goA6g0uvys4+eGscPu",

"Length":65

}:

    - /CGI/Security/sessionForget

This is used to tell the device that i need to find back the password. The device will return the salt and sessionID for next step “forgetCheck” to check.

Sample:

URL: http://192.168.1.108/CGI/Security/sessionForget?timestamp:1570762095422

Type: GET(only)

Return body:

| Json | Description | Value |
| --- | --- | --- |
| "salt" : "IZKsJIHKl594jnflh726M5c0sXQ8gbgA", | Used for next step /CGI/Security/SelfExt/forgetCheck. It is important. The server need this and the super password together Encrypted. |  |
| "Sessionid":"36Or1Co5H4o6HbyU" | The SessionId, Also used for next step CGI/Security/ SelfExt/forgetCheck. The body forgetCheck command has SessionID , should same as this. |  |
| "statusString": "OK", |  |  |
| "statusValue": 200 |  |  |

    - /CGI/Security/SelfExt/forgetCheck

Used to send the superpassword to the device, let the device re initialize the password. The encrypted is not give out. Only give out after our company’s consent.

Sample:

URL:  http://192.168.1.41/ CGI/Security/ SelfExt/forgetCheck

{"SuperPassword":"hPGjO4m3","Sessionid":"36Or1Co5H4o6HbyU","Time":"20191011"}

Type: Post(only)

| Json | Description | Value |
| --- | --- | --- |
| "SuperPassword":"hPGjO4m3" | The super password and the salt value that from sessionForget together use MD5 to encrypted |  |
| "Sessionid":"36Or1Co5H4o6HbyU" | The SessionId,from “sessionForget” |  |
| "Time":"20191011" | The Time of computer |  |

The Return value

| Json | Description | Value |
| --- | --- | --- |
| "Token": "36Or1Co5H4o6HbyUWBN39A3235478DIcQrR2TXRQyu5o9K2E", | This is very important, need this as a key to encryopted the send ,for next step. /CGI/Security/DeviceInitOption. Detail please another document. |  |
| "statusValue" : 200 | OK or not |  |

  - Ability

| Order | description | GET | PUT |
| --- | --- | --- | --- |
| abilitysystemfunction | System capability | √ |  |
| abilityencode | Coding ability | √ |  |
| abilitycomm | Like if GPS has | √ |  |
| abilityptz | PTZ protocols that the device support | √ |  |
| abilityuart | Uart protocols that the device support. | √ |  |
| abilitycamera | Used for camera, DVR will return {camera : null},means it is DVR. If IPC ,will show if support AF and so on. | √ |  |
| dvedesc | To get some device’s static informaiton, alarm in, out audio channels and so on. | √ |  |

    - /CGI/Ability/abilitysystemfunction

Get the system function ability. This API can only support get. For example “LossDetect” this key, if this value is false means the device can not support video loss detection. When video loss there will be no alarm. So ability means support or not support.

The reply body is:

| Json | Description | Value |
| --- | --- | --- |
| "SystemFunction": { |  |  |
| "AlarmFunction": { |  |  |
| "AlarmConfig": false, | Sensor alarm | true:Support the function false:the function is not Supported |
| "BlindDetect": true, | Video occlusion detection alarm | true: false: |
| "LossDetect": false, | Video loss detection alarm | true: false: |
| "MotionDetect": true, | Motion detection alarm | true: false: |
| "NetAbort": true, | Net error detection | true: false: |
| "NetAlarm": true, |  | true: false: |
| "NetIpConflict": true, | IP conflict alarm | true: false: |
| "StorageFailure": true, | Storage device access failure alarm | true: false: |
| "StorageLowSpace": true, | Storage device capacity insufficent alarm | true: false: |
| "StorageNotExist": true, | Storage device remove alarm | true: false: |
| "VideoAnalyze": false, | Video Analyze function | true: false: |
| "CrossLine": true, | Cross line alarm | true: false: |
| "RegionDetect": true | Region detect alarm | true: false: |
| }， |  |  |
| "EncodeFunction": { |  |  |
| "CombineStream": false, | Combine Stream function，No support | true: false: |
| "DoubleStream": true, | Double Stream function | true: false: |
| "SnapStream": false, | Snap Function | true: false: |
| "WaterMark": false, | WaterMark function, No support | true: false: |
| }， |  |  |
| "InputMethod": { |  |  |
| "NoSupportChinese": false | SupportChinese or not | true: false: |
| }, |  |  |
| "NetServerFunction": { |  |  |
| "Net3G": false, | 3G function | true: false: |
| "Net8021X": false, | 8021X function | true: false: |
| "NetANNIDDNS": false, |  | true: false: |
| "NetARSP": false, | ARSP function | true: false: |
| "NetAlarmCenter": false, | AlarmCenter | true: false: |
| "NetAutoUpgrade": false, | No support | true: false: |
| "NetDDNS": true, | DDNS | true: false: |
| "NetDHCP": true, | DHCP | true: false: |
| "NetDNS": true, | DNS | true: false: |
| "NetDropbox": false, | Dropbox : An cloud storyage,NO support | true: false: |
| "NetEmail": true, | Email | true: false: |
| "NetFTP": true, | FTP | true: false: |
| "NetGB28181": false, | GB28181,chinese public security standard | true: false: |
| "NetGoogleDrive": false, | GoogleDrive,:an public cloud storyage | true: false: |
| "NetIPFilter": true, | IPFilter: used to limit the blacklist | true: false: |
| "NetLTP2P": false, | LTP2P: our p2p function | true: false: |
| "NetLocalSdkPlatform": false, | No Support | true: false: |
| "NetMobile": false, | not need to care | true: false: |
| "NetMutliCast": false, |  | true: false: |
| "NetNTP": true, | NTP function | true: false: |
| "NetNat": false, | not need to care | true: false: |
| "NetPPPoE": false, | PPPoE | true: false: |
| "NetRTSP": true, | RTSP | true: false: |
| "NetRTSPS": true, | RTSPS | true: false: |
| "NetSNMP": false, | SNMP | true: false: |
| "InstaOnCloud": false, | InstaOnCloud | true: false: |
| "NetRTMP": true, | RTMP | true: false: |
| "NetRTMPS": true | RTMPS | true: false: |
| }, |  |  |
| "OtherFunction": { |  |  |
| "DownLoadPause": true, | No Suppor | true: false: |
| "ExtraSupportRecord": false, |  | true: false: |
| "SDsupportRecord": false, | IPC with SD card will use this | true: false: |
| "SupportCustomTitle": false, | IF the device support Custom Title or not | true: false: |
| "SupportSignalTitle": false, | IF the device support Signal Title or not | true: false: |
| "SupportIPV6": true, | IF the device support IPV6 or not | true: false: |
| "USBsupportRecord": false, | No Support | true: false: |
| "AudioInput": true, |  | true: false: |
| "AudioInMethod": true, |  | true: false: |
| "AudioOutput": true, |  | true: false: |
| "AudioOutMethod": true |  | true: false: |
| } |  |  |
| } |  |  |

    - /CGI/Ability/abilitycomm

Get the serial ability. Display supported serial protocol.

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "ComProtocol" : |  |  |
| [ |  |  |
| "NONE", |  |  |
| "GPS", |  |  |
| "General", |  |  |
| "InteractCmd", |  |  |
| "Ranging", |  |  |
| "Transparent" |  |  |
| ] |  |  |
| } |  |  |

    - /CGI/Ability/abilityptz

Get the PTZ ability. Display supported PTZ protocol.

| Json | Description | Value |
| --- | --- | --- |
| { "PTZProtocol" : |  |  |
| [ |  |  |
| "NONE", |  |  |
| "HD-FH", |  |  |
| "HD-HXST", |  |  |
| "HD-MY"， |  |  |
| "HD-NVP", |  |  |
| "HD-TVI", |  |  |
| "HY", |  |  |
| "LG", |  |  |
| "PANASONIC", |  |  |
| "PELCO-9750", |  |  |
| "PELCOASCII", |  |  |
| "PELCOD-RS" |  |  |
| ] |  |  |
| } |  |  |

    - /CGI/Ability/abilityuart

Get the RS485 ability. Display supported rs485 protocol.

The reply body is:

| Json | Description | Value |
| --- | --- | --- |
| { "UartProtocol" : |  |  |
| [ |  |  |
| "NONE", |  |  |
| "DaHua", |  |  |
| "DaHua2", |  |  |
| "HB5003", |  |  |
| "HanBang", |  |  |
| "Hikvision", |  |  |
| "InteractCmd" |  |  |
| .... |  |  |
| ] |  |  |
| } |  |  |

    - /CGI/Ability/abilitycamera

This is mainly for IPCamera, not DVR

The reply body is:

| Json | Description | Value |
| --- | --- | --- |
| { "Camera" : |  |  |
| { |  |  |
| "AFSupport":0 | Support AF or not | 0: means no support, 1 means support |
| "AISupport"：0, | Support AI or not,means perimeter alert and object missing or object left |  |
| "PTZSupport"：0, | Support PTZ or not |  |
| "Reserve":0, |  |  |
| "WDRSupport":0, | Support WDR or not |  |
| } |  |  |
| } |  |  |

    - /CGI/Ability/devdesc

This is the device main function information, Now  it just can be get , no set CGI, But can be set in the device. The mainly is the alarm in and alarm out channels.

The reply body is:

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "AlarmInChannel": 0, | Alarm in port of device | No change |
| "AlarmOutChannel": 0, | Alar out port of device | No change |
| "AudioInChannel": 4, | Audio in Channel | No change |
| "AutoBoot": false, | No care | No change |
| "CVBStoSPOT": 0, | Support spot or not | Can change |
| "EnableCVBS": 1, | If CVBS is enable | Can change |
| "EnableComm": true, | This means RS232 com | Can change |
| "EnablePlayDefault": true, | No care |  |
| "EnablePtz": true, | PTZ function is enable or not | Can change |
| "GUITheme": "General", | No care now |  |
| "PadType": 13, | The pad of device | Can be change if the pad is not right |
| "RemoteType": 0, | No care |  |
| "TrailDay": 0, | No care |  |
| "VideoFormat": 0, | No care here |  |
| "localPalyDefault": 4, | The default playback channel | Can change |
| "localPalyMax": 4 | The max channel that can play one time. | No change |
| } |  |  |

    - /CGI/Ability/abilityencode

Get the encode ability. Like the device total encode power, The main stream can support how many compression resolution, the Encode type,like H.264/H.265. Support audio encode or not.

| Sample | Description | Value |
| --- | --- | --- |
| {"EncodeCapability": { |  |  |
| "ChannelMaxSetSync": 0, | No care |  |
| "CombEncodeInfo": [} | This is for comb encode, now no care |  |
| "EncodeInfo": [ |  |  |
| NOTE:The next is the encode ability for main stream |  |  |
| { |  |  |
| "CompressionMask": "0x00000180", | Mask for compression, one bit for one type. 0x00000180 change to binary is “110000000”, so it means support the seven and eight number of compression type. See the table Table. CompressionType |  |
| "Enable": true, |  | true:support the function false:the function is not Supported |
| "HaveAudio": true, |  | true:support the function false:the function is not Supported |
| "ResolutionMask": "0x00004800", | Mask for Resolution.Change to binary is 0100 1000 0000 0000. So 11 and 14 is 1, means support D1 and 1080. See the Table. Resolution Type |  |
| "StreamType": "MainStream" | Mean is Main Stream |  |
| }, |  |  |
| NOTE:The next is the encode ability for extra stream 2 |  |  |
| { |  |  |
| "CompressionMask": "0x00000180", |  |  |
| "Enable": true, |  | true:support the function false:the function is not Supported |
| "HaveAudio": true, |  |  |
| "ResolutionMask": "0x00000029", |  |  |
| "StreamType": "ExtraStream2" | ExtraStream |  |
| }, |  |  |
| "ExImageSizePerChannel": [ | no use now |  |
| "0x00000029", | Channel 0 |  |
| "0x00000000", | Channel 1 |  |
| "0x00000000" | ... |  |
| ], |  |  |
| "ExImageSizePerChannelEx": [ | This means mask of substream resolution when main stream is. |  |
| [ |  |  |
| "0x00000000", | The first line, and in Table. Resolution Type , the first resolution is D1, the value is 0x00000000, means if main stream is D1, sub stream support none resolution. |  |
| "0x00000000", |  |  |
| "0x00000000", |  |  |
| "0x00000000", |  |  |
| "0x00000000", |  |  |
| "0x00000000", |  |  |
| "0x00000000", |  |  |
| "0x00000000", |  |  |
| "0x00000000", |  |  |
| "0x00000000", |  |  |
| "0x00000000", |  |  |
| "0x00000029", | This is the eleven line, so means when main stream is 720P, ths substream resolution mask is “0x00000029”=0010 1001， and mean sub stream should support D1 CIF and VGA |  |
| "0x00000000", |  |  |
| "0x00000000", |  |  |
| "0x00000029", |  |  |
| "0x00000000", |  |  |
| "0x00000000", |  |  |
| "0x00000000", |  |  |
| "0x00000000", |  |  |
| "0x00000000", |  |  |
| "0x00000000" |  |  |
| ], |  |  |
| "MinBitrate": 128, | Bit rate is fixed at 128. |  |
| "MaxBitrate": 49152, | The bitrate should be less then this |  |
| "MinSubBitrate": 128, | sub stream Bit rate is fixed at 128. |  |
| "MaxSubBitrate": 49152, | sub stream The bitrate should be less then this |  |
| "MaxEncodePower": 49582080, ; | Means the total encode power, so when the main stream FPS lower, the substream FPS can be higher. MaxEncodePower = Mainstream power(main max Resolution * FPS) + subStream power(sub max Resolution * FPS) |  |

The reply body is:

| Table. Resolution Type |  |  |
| --- | --- | --- |
| Resolution | W*H PAL | NTSC |
| D1 | 704x576 | 704x480 |
| HD1 | 704x288 | 704x240 |
| 1080N | 944x1080 | 944x1080 |
| CIF | 352x288 | 352x240 |
| QCIF | 176x144 | 176x120 |
| VGA | 640x480 | 640x480 |
| QVGA | 320x240 | 320x240 |
| H3M | 960x1440 | 960x1440 |
| H4M | 1280x1440 | 1280x1440 |
| ND1 | 240x192 | 240x192 |
| 960H | 928x576 | 928x576 |
| 720P | 1280x720 | 1280x720 |
| 960P | 1280x960 | 1280x960 |
| UXGA | 1600x1200 | 1600x1200 |
| 1080P | 1920x1080 | 1920x1080 |
| WUXKA | 1920x1200 | 1920x1200 |
| 4M | 2560x1440 | 2560x1440 |
| 3M | 2304x1296 | 2304x1296 |
| 5M | 2592x1944 | 2592x1944 |
| 6M | 3072x2048 | 3072x2048 |
| 8M | 3840x2160 | 3840x2160 |
| H5M | 1296x1944 | 1296x1944 |
| 4K | 4096x2160 | 4096x2160 |
| 8K | 7680x4320 | 7680x4320 |
| 2560x1536 | 2560x1536 | 2560x1536 |
| 2560x1920 | 2560x1920 | 2560x1920 |
| 2880x1620 | 2880x1620 | 2880x1620 |
| 360P | 640*360 | 640X360 |

| - Table. CompressionType |  |
| --- | --- |
| CompressionType | Enum Number |
| DIVX MPEG4 | 0 |
| MS MPEG4 | 1 |
| MPEG2 | 2 |
| MPEG1 | 3 |
| H.263 | 4 |
| MJPG | 5 |
| FCC MPEG4 | 6 |
| H.264 | 7 |
| H.265 | 8 |
| H.264+ | 9 |
| H.265+ | 10 |

Table. Stream Code Value

Every resolution the bit rate is different on each video quality, at the total Frames. When FPS change , need to use Function to get. The Function is {512, 768, 1024, 1536, 2048, 2560}, ///< D1 when resolution is D1 {384, 512, 768, 1024, 1536, 2048}, ///< HD1 {1024,1536, 2048, 2560, 3072, 4096},///< H1080P {64, 192, 384, 512, 768, 1024}, ///< CIF {64, 128, 192, 384, 448, 512}, ///< QCIF {512, 768, 896, 1280, 1536, 2048}, ///< VGA {256, 384, 512, 640, 768, 1024}, ///< QVGA {1024, 2048, 3072, 4096, 6144, 8192}, ///< H3M {1024, 2048, 3072, 4096, 6144, 8192}, ///< H4M {64, 128, 256, 448, 512, 768}, ///< ND1 {896, 1024, 1536, 2048, 3072, 4096}, ///< 650TVL {665, 998, 1331, 1664, 1996, 2662}, ///< 720P {665, 998, 1331, 1664, 1996, 2662}, ///< 1_3M {665, 998, 1331, 1664, 1996, 2662}, ///< UXGA {665,1331, 1996, 2662, 3993, 5324}, ///< 1080P {665,1331, 1996, 2662, 3993, 5324}, ///< WUXGA {665,1331, 1996, 2662, 3993, 5324}, ///< 4M {665,1331, 1996, 2662, 3993, 5324}, ///< 3M {665,1331, 1996, 2662, 3993, 5324}, ///< 5M {1024, 2048, 3072, 4096, 6144, 8192}, ///< 6M {1024, 2048, 3072, 4096, 6144, 8192}, ///< 8M For bi

    - /CGI/Ability/abilityencodefps

Sometimes , the encodeability is not use enough. Like if the 5M can support to 15FPS , if use encode abilityencode, the 4M maybe can up to 20FPS. But actually can also only support to 15fps. So we add abilityencodefps to solve the problem.

| Sample | Description | Value |
| --- | --- | --- |
| {"EncodeFpsAbility": [{"FpsInfo": [ |  |  |
| { |  |  |
| "MainFpsInfo": { | The mainstream fps information |  |
| "Height": 1080, | height |  |
| "ImgSize": 14, // no care about this |  |  |
| "MaxFps": 30, | The max fps ,at this resolution can support. |  |
| "Width": 1920 |  |  |
| }, |  |  |
| "SubFpsInfo": [ | The is sub FPS info. In the main stream resolution how many substream resoltuion support then will reply every section. |  |
| { |  |  |
| "Height": 480, |  |  |
| "ImgSize": 5, |  |  |
| "MaxFps": 30, |  |  |
| "Width": 640 |  |  |
| }, |  |  |
| { "Height": 288, "ImgSize": 3, "MaxFps": 30, "Width": 352 }, |  |  |
| { "Height": 0, "ImgSize": 0, "MaxFps": 0, "Width": 0 }, | If all is zero, no care |  |
| } |  |  |

  - General Configure

Note: For configure API, every one i will give out the body, If for “GET” operation do not need to send the body. And for “PUT” operation  need to send the body. If some field write “no care”, “do not change” and so on, please send back the value as get.

| Command | Description | GET | PUT | POST |
| --- | --- | --- | --- | --- |
| general | General configuration | √ | √ |  |
| location | Local configuration | √ | √ |  |
| ntp | Time zone configuration | √ | √ |  |
| time | Current time | √ | √ |  |
| autoMaintainConfig | Automatic maintenance | √ | √ |  |
| initgeneral |  | √ |  | √ |

    - /CGI/General/initgeneral

GET/POST  Language and time information. This is only happen in first login of device after initialization.

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "DateFormat": "YYMMDD", |  | "YYMMDD" "MMDDYY" "DDMMYY" |
| "Language": "English", | The language default device is. | Through support language can get how much language supported. See /CGI/General/supportlanguage |
| "SystemTime": "2019-09-26 13:52:59",, | The system time is . |  |
| "TimeZone": 0, | Time zone | See table GMT |
| "VideoFormat": "PAL" | not use now |  |
| } |  |  |

    - /CGI/General/supportlanguage

Get the language that device supported.

Json

"Language": [ "Arabic", "Brazilian", "Bulgarian", "Czech", "Dutch", "Englis…inese", "Slovakia", "Spanish", "Swedish", "Thai", "TradChinese", "Turkey" ]}

    - /CGI/General/general

GET/PUT General configuration.

Web Page: System Config->System Config->System Config

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "AutoLogout": 7, | The login out time for menu [0, 120], no use for IPC | [0,120] |
| "LocalNo":0, | The Number of Local machine [0, 998], No Use | [0,998] |
| "MachineName":"LocalHost", | The Local Machine Name |  |
| "OverWrite": "OverWrite", | OverWrite or Stop Record when HDD full |  |
| "ScreenAutoShutdown": 10, | not use now |  |
| "ScreenSaveTime": 0, | not use now |  |
| "SnapInterval": 2, | Snap time interval，need device support |  |
| } |  |  |

    - /CGI/General/location

GET/PUT local general configuration, mainly summertime, video format,and language

Web Page: System Config->System Config->General Config

For get the reply body is the JSON value in the table. And for PUT the send body is te JSON value int the table.

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "DSTEnd": { | Summer time end date |  |
| "Day": 1, |  | [0,998] |
| "Hour": 1, |  |  |
| "Minute": 1, |  |  |
| "Month": 1, |  |  |
| "Week": 1, |  |  |
| "Year": 2000 |  |  |
| }, |  |  |
| "DSTRule": "off", | Summer time rule off or on | on:open the function off:close the function |
| "DSTStart": { |  |  |
| "Day": 1, |  |  |
| "Hour": 1, |  |  |
| "Minute": 1, |  |  |
| "Month": 1, |  |  |
| "Week": 1, |  |  |
| "Year": 2000 |  |  |
| }, |  |  |
| "DateFormat": "YYMMDD", | DateFormat | "YYMMDD" "MMDDYY" "DDMMYY" |
| "DateSeparator":"-", | DateSeperator | "-" "." "/" |
| "Language":"English", | no use |  |
| "TimeFormat":"24", | Time Format | "24" "12" |
| "VideoFormat":"PAL", | VideoFormat “PAL” | “PAL” “NTSC” |
| "WorkDay": 62 |  |  |
| } |  |  |

    - /CGI/General/coordinate

GET/PUT Equipment longitude and latitude information

For get the reply body is the JSON value in the table. And for PUT the send body is te JSON value int the table.

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Latitude": { |  |  |
| "Degree": 1, | Latitude,0-90 | int |
| "Minute": 0, | 0-59 | int |
| "Second": 0.00, | 0<= Minute < 60 | double |
| "Direction": "N", | N , S | char |
| }, |  |  |
| "Longitude": { |  |  |
| "Degree": 1, | 0-180 |  |
| "Minute": 0, | 0-59 |  |
| "Second": 0.00, | 0<= Minute < 60 |  |
| "Direction": "E" | E,W |  |
| }, |  |  |
| } |  |  |

    - /CGI/General/ntp

Get/PUT Time Zone Configuration

Web Page: System Config->System Config->NTP Setting

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Enable":false, | Open or close function | true:open the function false:close the function |
| "Server": { |  |  |
| "Address":"0x00000000", | Please do not change |  |
| "Anonymity":false, | Please do not change,Server is a normal structure, so some field will no use in some place. |  |
| "Name": time.windows.com", | NTP server |  |
| "Password":"", | Please do not change no use |  |
| "Port":123, | NTP server port |  |
| "UserName":"" | Please do not change no use |  |
| }, |  |  |
| "TimeZone":13, | See the next table. |  |
| "UpdatePeriod":10 | How long to change the time |  |
| } |  |  |

| Table GMT |  |
| --- | --- |
| 0 | (GMT+00:00) Dublin, Edinburgh, London |
| 1 | (GMT+01:00) Amsterdam, Berlin, Rome, Paris |
| 2 | (GMT+02:00) Athens, Jerusalem, Istanbul |
| 3 | (GMT+03:00) Baghdad, Kuwait, Moscow |
| 4 | (GMT+03:30) Tehran |
| 5 | (GMT+04:00) Caucasus standard time |
| 6 | (GMT+04:30) Kabul |
| 7 | (GMT+05:00) Islamabad, Karachi, Tashkent |
| 8 | (GMT+05:30) Madras, Mumbai, New Delhi |
| 9 | (GMT+05:45) Kathmandu |
| 10 | (GMT+06:00) Almaty, Novosibirsk, Dhaka |
| 11 | (GMT+06:30) Yangon |
| 12 | (GMT+07:00) Bangkok, Hanoi, Jakarta |
| 13 | (GMT+08:00) Beijing, Urumqi, Singapore |
| 14 | (GMT+09:00) Seoul, Tokyo, Osaka, Sapporo |
| 15 | (GMT+09:30) Adelaide, Darwin |
| 16 | (GMT+10:00) Melbourne, Sydney, Canberra |
| 17 | (GMT+11:00) Magadan, Solomon Islands |
| 18 | (GMT+12:00) Auckland, Wellington |
| 19 | (GMT+13:00) Nuku'alofa |
| 20 | (GMT-01:00) Cape Verde Islands, Azores |
| 21 | (GMT-02:00) Central Atlantic |
| 22 | (GMT-03:00) Georgetown, Brasilia |
| 23 | (GMT-03:30) Newfoundland |
| 24 | (GMT-04:00) Atlantic Time (Canada) |
| 25 | (GMT-05:00) Eastern Time (U.S. and Canada) |
| 26 | (GMT-06:00) Central Time (US and Canada) |
| 27 | (GMT-07:00) Mountain time (U.S. and Canada) |
| 28 | (GMT-08:00) Pacific Time (U.S. and Canada) |
| 29 | (GMT-09:00) Alaska |
| 30 | (GMT-10:00) Hawaii |
| 31 | (GMT-11:00) Midway, Samoa |
| 32 | (GMT-12:00) Day line west |

    - /CGI/General/time

GET/PUT local time:

Web Page: System Config->System Config->NTP Setting

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "SystemTime": "2018-11-01 19:07:36", | System Time |  |
| "iDateFormat": 0, | Date format | 0:YYMMDD 1:MMDDYY 2: DDMMYY |
| "iDateSeparator": 0 | Date separator symbol | 0:“-” 1:“/” 2:“ ” |
| } |  |  |

    - /CGI/General/autoMaintainConfig

GET/PUT AutoMaintain configuration

Web Page: System Config->System Tool->Auto Maintain

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "AutoDeleteFilesDays": 0, | How many days to delete record file. | [0-100] |
| "AutoRebootDay":"Tuesday", | Auto Reboot day. | Never Everyday Sunday Monday Tuesday Wednesday Thursday Friday Saturday |
| "AutoRebootHour": 1 | The time to reboot. | [0-23] and 0 means 12:00 AM |
| } |  |  |

    - /CGI/Custom/comm

GET/PUT comm configuration

Web Page: System Config->System Tool->Serial port

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Attribute": [ |  |  |
| 115200, | Baud rate |  |
| "NONE", | check | "None" "Odd" "Even" check value "Mark" "Space" |
| 8, | Data bit |  |
| 1 | Stop bit |  |
| ], |  |  |
| "PortNo": 1, | Don’t used |  |
| "ProtocolName": "NONE" | Serial port function |  |
| }, |  |  |
| { |  |  |
| ... |  |  |

    - /CGI/PTZ/rs485/channels/<channelNo>

GET/PUT rs485 configuration.

Web Page: System Config->System Tool->PTZ

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Attribute": [ |  |  |
| 9600, | Baud rate |  |
| "NONE", | check | "None" "Odd" "Even" check value "Mark" "Space" |
| 8, | Data bit |  |
| 1 | Stop bit |  |
| ], |  |  |
| "DeviceNo": 1, | address |  |
| "NumberInMatrixs": 0, |  |  |
| "PortNo": 1, | Don’t used |  |
| "ProtocolName": "NONE" | protocol function |  |
| } |  |  |

    - /CGI/PTZ/ptzcfg/channels/0

GET/PUT PTZ configuration.

Web Page: System Config->System Tool->PTZ

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Attribute": [ |  |  |
| 9600, | Baud rate |  |
| "NONE", | check | "None" "Odd" "Even" check value "Mark" "Space" |
| 8, | Data bit |  |
| 1 | Stop bit |  |
| ], |  |  |
| "DeviceNo": 1, | address |  |
| "NumberInMatrixs": 0, |  |  |
| "PortNo": 1, | Control mode |  |
| "ProtocolName": "PELCOD" | protocol function |  |
| } |  |  |

    - /CGI/General/guiset

GET/PUT guiset configuration.

Web Page: System Config->System Tool->OutPutMode

| Json | Description | Value |
| --- | --- | --- |
| { |  | { |
| "AlarmStateEnable": true, |  |  |
| "ChanStateBitRateEnable": true, |  |  |
| "ChanStateLckEnable": true, |  |  |
| "ChanStateMtdEnable": true, |  |  |
| "ChanStateVlsEnable": true, |  |  |
| "ChannelTitleEnable":true, |  |  |
| "RecordStateEnable": true, |  |  |
| "TimeTitleEnable": true, |  |  |
| "WindowAlpha": 175, |  | 128-255 |
| "RemoteEnable": true, |  |  |
| "Deflick": true, |  |  |
| "CarInfo": flase, |  |  |
| "GPSInfo": true |  |  |
| } |  |  |

    - /CGI/General/videoout

GET/PUT videoout configuration.

Web Page: System Config->System Tool->OutPutMode

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Resolution":[ | Resolution |  |
| .... |  |  |
| .... |  |  |
| .... |  |  |
| ] |  |  |
| "CurIndex": 1 | Current selection |  |
| } |  |  |

    - /CGI/General/audioinout

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "AudioInEnable": true, | audio input | false: close the function true: open the function |
| "AudioInExternal": false, | Audio input external connection method | false: Internal true: external Now out of use |
| "AudioOutEnable": true, | audio output | false: close the function true: open the function |
| "AudioOutExternal": false | Audio output external connection method | false: Internal true: external Now out of use |
| "AudioInVolumn": 50 | Input volume size | int 0 - 100 |
| "AudioInEncodeType":"G.711U" | input Encode Format | string support the following: G.711U |
| "AudioInMode": "Internal Connection" | Audio input connection method | string |
| "AudioOutMode": "Internal Connection" | Audio output connection method | string |
| AudioInList: [ "Internal Connection", "External Connection" ] | List of sound input methods |  |
| AudioOutList: [ "Internal Connection", "External Connection" ] | List of sound output methods |  |
| } |  |  |

    - /CGI/General/basicinfo

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "ProductModel": "IPC3624LE-ADF28K-WP@@", | Product Model |  |
| "SoftwareVersion": "GIPC-B6215.1.103.241230", | Software Version |  |
| "HardwareVersion": "A", | Hardware Version |  |
| "BootVersion": "V4.0", | Boot Version |  |
| "SerialNumber": "210235XMVN3252004106", | Serial Number |  |
| "SystemTime": "2025/7/24 16:52:34", | Current device time |  |
| "Uptime": "3day.5hour.1min", | Equipment operating duration |  |
| } |  |  |

    - /CGI/General/security

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "OnvifAuth":1, | onvif Authentication | int 0：NONE 2：Digest |
| "RtspAuth": 0, | rtsp Authentication | int 0：NONE 2：Digest |
| "HttpAuth": 0 | http Authentication | int 0：NONE 2：Digest |
| } |  |  |

    - /CGI/General/reboot

After logging in, you can directly request to restart

    - /CGI/General/OneClickrestore

After logging in, you can directly request to restore

    - /CGI/General/restore

PUT restore configuration

It will only take effect after the system is restarted.

| Json | Description | Value |
| --- | --- | --- |
| { |  | { |
| "NetCommon": 1, |  |  |
| "Account": 1, |  |  |
| "General": 1, |  |  |
| "Encode": 1, |  |  |
| "Record": 1, |  |  |
| "CommPtz":1, |  |  |
| "NetServer": 1, |  |  |
| "Alarm": 1, |  |  |
| "Preview": 1, |  |  |
| "CameraPARAM": { |  |  |
| "VideoColor": 1, | Image color config |  |
| "VideoColorEx": 1, | Image color config, expansion |  |
| "CameraAdd": 1, | image param |  |
| "CameraIsp": 1, | ISP param |  |
| "Cameraprofile": 1, |  |  |
| "FaceSnap": 1, |  |  |
| "SupplyLight": 1, |  |  |
| "WhiteBalance": 1, |  |  |
| } |  |  |
| } |  |  |

    - /CGI/General/default

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Name": "Camera.Param" |  | "Detect.BlindDetect.[0]", "Detect.RegionDetect.[0]", "Detect.CrossLineDetect.[0]", "Detect.MotionDetect.[0]", "Detect.AlarmIn.[0]", "Detect.AlarmOut", "AVEnc.Encode.[0]", "AVEnc.Snap.[0]", "AVEnc.VideoColor", "Camera.Param", "Camera.IspParam", "Camera.WBparam", "Camera.Profile", "NetWork.NetFTP", "NetWork.NetEmail", "NetWork.MTU", "NetWork.NetCommon", "NetWork.NetDHCP", "NetWork.NetDNS", "NetWork.NetDDNS", "NetWork.Upnp", "NetWork.LibRtmp", "NetWork.RTSP", "NetWork.DAHUATCP", "NetWork.NetHttps", "FTP.Path", "FACESNAP", "Camera.SupplyLight", "NetWork.CustomSnap", "NetWork.Gb28181", |
| } |  |  |

  - Encode

Note: For configure API, every one i will give out the body, If for “GET” operation do not need to send the body. And for “PUT” operation  need to send the body. If some field write “no care”, “do not change” and so on, please send back the value as get.

| Command | Description | GET | PUT |
| --- | --- | --- | --- |
| encode/channels/<channelNo> | Single channel encoding configuration | √ | √ |

    - /CGI/Encode/encode/channels/<channelNo>

GET/PUT Encode configuration. For IPC http://<IP>/CGI/Encode/encode/channels/0,Also please see the ability of encode.  /CGI/Ability/abilityencode
Web Page:Camera->Encode Config->Encode Setting

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "ExtraFormat": [ | SubStream Audio and video encoding format. |  |
| { |  |  |
| "Audio": { | Audio Config, Please do not change the value |  |
| "BitRate": 10, |  |  |
| "MaxVolume": 10, |  |  |
| "SampleRate": 10 |  |  |
| }, |  |  |
| "AudioEnable": true, | Enable or Disable Audio | true:open the function false:close the function |
| "Video": { | Video Format |  |
| "BitRate": 388, | Bit Rate of Video |  |
| "BitRateControl": "VBR", | VBR or CBR, | VBR:Pic quality stable and bit rate will change. CBR:Bit rate stable and pic quality will change |
| "Compression": "H.264", | Encoding type | MJPEG H.264 H.264+ H.265 H.265+ |
| "FPS": 15, | Frame rate |  |
| "GOP": 2, |  |  |
| "Quality": 3, | Video Quality | [1-6]1: bad 6:Excellent |
| "Resolution": "CIF" | Video Resolution See the Ta |  |
| }, |  |  |
| "VideoEnable": true |  |  |
| }, |  |  |
| { | Do not care this type, For Substream just the first is useful |  |
| "Audio": { |  |  |
| ... |  |  |
| }, |  |  |
| "AudioEnable": false, |  |  |
| "Video": { |  |  |
| ... |  |  |
| }, |  |  |
| "VideoEnable": false |  |  |
| }, |  |  |
| ], |  |  |
| "MainFormat": [ | Main Stream Audio and Video Format. The parameter is same as sub stream |  |
| { |  |  |
| "Audio": { |  |  |
| ... |  |  |
| }, |  |  |
| "AudioEnable": true, |  |  |
| "Video": { |  |  |
| .... |  |  |
| }, |  |  |
| "VideoEnable": true |  |  |
| }, |  |  |
| ], |  |  |
| "SnapFormat": [ | Please do not change this, this stream can not be change |  |
| {, |  |  |
| "Audio": { |  |  |
| ..... |  |  |
| }, |  |  |
| "AudioEnable": false, |  |  |
| "Video": { |  |  |
| ..... |  |  |
| }, |  |  |
| "VideoEnable": true |  |  |
| }, |  |  |
| ] |  |  |
| } |  |  |

Reference:

/CGI/Ability/abilityencode

  - Net Service API Functions

Note: For configure API, every one i will give out the body, If for “GET” operation do not need to send the body. And for “PUT” operation  need to send the body. If some field write “no care”, “do not change” and so on, please send back the value as get.

In the later API, some body has the field “password”, so these text is more important, and can not be seen by other. We do encrypt the whole JSON body for safe.

| Command | Description | GET | PUT |
| --- | --- | --- | --- |
| ipfilter | IP permissions configuration | √ | √ |
| ddns | DDNS configuration | √ | √ |
| email | E-MAIL configuration | √ | √ |
| pppoe | PPPOE configuration | √ | √ |
| upnp | UPNP configuration | √ | √ |
| arsp | ARSP configuration | √ | √ |
| cloud | Cloud configuration | √ | √ |
| ftp | FTP configuration | √ | √ |
| rtsp | RTSP configuration | √ | √ |
| https | HTTPS configuration, The whole function has 1: Get / PUT https configuration 2: Download the certificate 3: Get the certificate information 4: Delete the certificate | √ | √ |

Reference:

/CGI/Ability/abilitysystemfunction

The ability of  “NetServerFunction”

    - /CGI/NetService/ipfilter

GET/PUT  ip permissions

Web Page: Network Config->Net Service->IPfilter

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Banned": [ | The banned IP address |  |
| "0x3301A8C0", | 192.168.1.51 | C0=192 A8=168 01=1 33=51 |
| ...... |  |  |
| ...... |  |  |
| ], |  |  |
| "Enable": false, | Open or close function | true:open the function false:close the function |
| "Trusted": [ | The trusted ip address |  |
| "0x00000000", |  |  |
| "0x00000000", |  |  |
| ...... |  |  |
| ...... |  |  |
| } |  |  |

Reference:
/CGI/Ability/abilitysystemfunction

    - /CGI/NetService/ddns

GET/PUT DDNS Configuration:

Web Page: Network Config->Net Service->IPfilter

| Json | Description | Value |
| --- | --- | --- |
| [ |  |  |
| { |  |  |
| "DDNSKey": "Topsddns", | DDNS type as an key, you can not change |  |
| "Enable": false, | you can only set one type to be true.Means if you set Topsddns to true, then the Oray must be false | true false |
| "HostName": "your.topsddns.net", |  |  |
| "Server": { |  |  |
| "Address": "0x0100060A", | The server address,but please do not change |  |
| "Anonymity": false, |  |  |
| "Name": "topsddns.net", | The real DDNS domain name, please do not change |  |
| "Password":"", | The password of the DDNS account |  |
| "Port": 80, |  |  |
| "UserName": "" | User name of the DDNS account |  |
| } |  |  |
| }, |  |  |
| } |  |  |
| { |  |  |
| "DDNSKey": "Oray", |  |  |
| "Enable": false, |  |  |
| "HostName": "your.gicp.net", |  |  |
| "Server": { |  |  |
| }, |  |  |
| ] |  |  |

    - /CGI/NetService/ddnsstatus

GET ddnsstatus Configuration. Only support Get.

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| "DdnsType":"CP Plus DDNS - Free", |  |  |
| "Online":true, | True: Connected False: Disconnect | true false |
| "StatusCode":-1, | -1: connet server failed. -2: unknown error. -3: Delete domain name: unknown error. -4: Update domain name: unknown error. -5: Detect domain name: unknown error. 901: Invalid domain name 902: The MAC ID is not valid 903: The MAC ID is not allowed 904: Invalid IP Address 905: Invalid Device Code 906: Invalid OEM ID 907: The Domain name already exists but belongs to some other MAC Address 908: The Domain name already exists and belongs to same MAC Address 909: The MAC Address already exists and is mapped to different Domain name. Record updated and mapping set to new Domain name 910: The Domain name created successfully 911: The Domain name could not be created for unknown reasons 912: Server temporarily offline 701: No record found 801: Update success 601: Record successfully deleted | int |
| } |  |  |

    - /CGI/NetService/email

GET/PUT E-mail Configuration

Web Page: Network Config->Net Service->Email

For email these configure has password, for safe the whole will be encrypted.  The whole JSON will use DES + BASE64 to encrypt.

For example:

The encrypted

{

"Content" : "eNjFW93aIR12JgDIZv/e5izmWCghdR1CTh/8Si/wyIKaFVqPrrMUDAGzIoUoa+YiP0Xf5Qk3y2taD6Pdp/+baJfZyIuTiIatiT27/2UDNk1rZ4L4URV6rc7GzPZTdpwyQYeRnCYGNaFeE933oQgPHTL+rtfEIejuf7CF/8Q5tcWd2jCs5RFyVTrhdgHjbYrUO/fvSokftV/QLbukcB9IUPF/9v6OmjBbi3bFtr4g559PmjJRD82KuuZlm6GFZI3vVHsP0RrtCfKkWK7MASgOYjTBtH9atyBTi3bFtr4g55+yy8VA02fbB5Q6FH/JQBA/lDoUf8lAED+UOhR/yUAQP7BkQD7bsRwKX9D/sSFQolVRNOsneU3iZOX+fai21PDiiJbgAL47XrSrzHSsLxcvVC/5h5BUccAFk/Ekl3T4Txgva9Z/wGfbQLWu3M4tj60af7CF/8Q5tcVUTumbAo5Re9Fkf2K7v6tP9n4g09uuRIzPscnjtn0rN7/ClsURIKyp/yOyGcK17fVstQiPpYhO6Ts4WSRn113K", "Length" : 544

}

| JSON | Description | Value |
| --- | --- | --- |
| { |  |  |
| “Attachment”: 0 | Add attachment or not | 0 ,1 |
| "Enable": true, | Enable Email or not | true false |
| "HealthEnable": 0, | Enable test function or not |  |
| "Interval": 0, | The health mail send time. | [1-1440]Minutes |
| "MailServer": { |  |  |
| "Address": "0x00000000", | Here is no use |  |
| "Anonymity": false, | No use now |  |
| "Name": "smtp.gmail.com", | SMTP server name |  |
| "Password": "", | The password of sender email account. | DES+ BASE64 |
| "Port": 465, | SMTP Port |  |
| "UserName": "thusdfn@126.com" | The email account |  |
| }, |  |  |
| "Port": 25, |  |  |
| "Recievers": ["zb1208@126.com", "none", "none", "none", "none"], | receiver email account |  |
| "Schedule": ["0 00:00:00-24:00:00", "0 00:00:00-24:00:00"] | Design for time send email, but now is no use, please do not change |  |
| "SendAddr": "zhubo@tops.com", | sender name |  |
| "Title": "Alarm Message", |  |  |
| "UseSSL": true | USE SSL or no | true false |
| } |  |  |

    - /CGI/NetService/emailtest

PUT, This command is send to the device and let device send email to test if the email username of password is right or not.

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "StatusCode" : 0, |  | 0:Successful mail delivery -1:Faile -2:Error in mailbox login username -3:Error password for mailbox login -4:Unable to connect to the server |
| "statusString" : "Unable to connect to the server" |  |  |
| } |  |  |

    - /CGI/NetService/upnp

GET/PUT UPNP configuration

Web Page: Network Config->Net Service->Upnp

This type do not need encrypt.

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Enable": false, | Enable UPNP or Disable | true:open the function false:close the function |
| "HTTPPort": 81, | The Value can not be changed by handle, IPC will show auto port for HTTP |  |
| "MediaPort":34567, | Port for media, OWN protocol. Can not change also. |  |
| "MobilePort": 34599, | Port for mobile,Can not change by handle also |  |
| "State": false | no use now |  |
| } |  |  |

    - /CGI/NetPlatform/cloud

GET/PUT Cloud configuration

Web Page: Network Config->Net Service->P2P

This type do not need encrypt.

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Enable": 1, | Enable Cloud or not | 0:close the function 1:open the function |
| "Port": 34600, | The Mobil Port |  |
| "Server": "www.topslink.net", | The Server address, you can watch video throw the web server. |  |
| "UID": "rjksi8h0e6ut" | cloud ID, just show ,no change |  |
| } |  |  |

    - /CGI/NetService/ftp

GET/PUT  FTP configuration

Web Page:  Network Config->Net Service->FTP

This type need to be encrypted.

| JSON | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Directory": "", | Remote Directory |  |
| "Enable": false, | Open or close function | true:open the function false:close the function |
| "MaxFileLen":128, | Max File Len of the file, UNIT: MB |  |
| "Server": { | The FTP server info |  |
| "Address":"0x00000000", | The Server address |  |
| "Anonymity":false, | Anonymity |  |
| "Name":"FTP", | Please do not use, ever time the name will be created by device self, so do not change |  |
| "Password":"", |  |  |
| "Port": 21, |  |  |
| "UserName": "" |  |  |
| }, |  |  |
| "TimeSection": [ | Time set to use FTP to upload, 7days and every day has 6 sector to set. But now no use, Please do not change |  |
| ] |  |  |
| ] |  |  |
| } |  |  |

    - /CGI/NetService/ftpPath

GET/PUT  FtpPath configuration

Web Page:  Record->Destination->FtpPath

| JSON | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Emergency": 0, |  |  |
| "FtpPath": { | Record |  |
| "dwloacl":"0x0", | The dwloacl field represents timing, motion check, alarm |  |
| "dwFTP": "0x0", | DwFTP field represents timing,motion check,alarm |  |
| "dwNAS":"0x0", | DwNAS fields represent timing, motion check, alarm |  |
| } |  |  |
| {......} | Ditto Snapshot |  |
| "Reserve":"", |  |  |
| } |  |  |

    - /CGI/NetService/ftptest

PUT  Command to check if the FTP configuration is right or not.

Web page: Record->Destination->Ftp

The encrypted

{

"RequestURL" : "/CGI/NetService/ftptest", "statusCode" : 2, //if success return 1 "statusString" : "Device Error". //if success return OK

}

    - /CGI/NetService/pppoe

GET/PUT RTSP configuration.

Web Page:  Network Config->Net Service->PPPoE

The encrypted

{

"Content": "eNjFW93aIR2S2iGRsKUnugPHNcqHWVxaL9ejZvXi6uxc+gcrEfSNrzIJXLzPuCIZ2B9I4asYXy7brWUf9YoxVB/k5IsAa/6cMas/dNrRgXZeA680FHFu4Cs6GN5mokFqKJVB4oUiZk+u6la0KO4wB2xMpst36LzGjX6dXFPSne6GGWdgReR4X7CZGnNi6LvwxTH2r9FS0i4ANZwFeVmkyhA07Kjjq4//T+ff0FFxEeA=",

"Length": 236

}

| JSON | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Enable": false, | Open/close the function | true:open the function false:close the function |
| "HostIP": "0x2901A8C0", | The PPPOE IP address, created by device, can not change by handle. | C0=192 A8=168 01=1 29=41 |
| "Server": { |  |  |
| "Address": "0x00000000", | ///< no use |  |
| "Anonymity": false, | ///< no use |  |
| "Name": "PPPoE", | ///< no use |  |
| "Password": "", | PPPOE password | DES+BASE64 |
| "Port": 1900, | no use |  |
| "UserName": "afaf" | PPPOE username |  |
| } |  |  |
| } |  |  |

Original text body:

    - /CGI/NetService/rtsp

GET/PUT RTSP configuration.

Web Page:  Network Config->Net Service->RTSP
So like pppoe, here also need encrypt the whole JSON text.

| JSON | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Client": { | For IPC is no use ,because IPC is for server |  |
| "Address": "0x00000000", |  |  |
| "Anonymity": false, |  |  |
| "Name": "", |  |  |
| "Password": "", |  |  |
| "Port": 554, |  |  |
| "UserName": "" |  |  |
| }, |  |  |
| "IsClient": false, |  |  |
| "IsServer": true, | PC should be server mode | true:open the function false:close the function |
| "Server": { |  |  |
| "Address": "0x00000000", | no use |  |
| "Anonymity": false, | no use |  |
| "Name": "", | no use |  |
| "Password": "", | no use |  |
| "Port": 554, | RTSP server port |  |
| "UserName": "", | no use |  |
| "RtspsEnable": false, | true: Enable function; false: Disable function |  |
| "RTSPSPort": 322, | RTSPS server port |  |
| }, |  |  |
| "Multicast": { |  |  |
| "MainAddress": "0x010000EF", | Mainstream video multicast address |  |
| "MainPort": "1234", | Mainstream video multicast port |  |
| "SubAddress": "0x020000EF", | Substream video multicast address |  |
| "SubPort": "1235" | Substream video multicast port |  |
| } |  |  |
| } |  |  |

    - /CGI/NetService/https

GET/PUT HPPTS configuration.

For the whole https configure can seperate to 3 steps:
1: 	Get https configure

2: 	Create the certificate.  The create function will do in the device. Just send the information to the device.

3:   Install the cettificate

Web Page:  Network Config->Net Service->HTTPS

| JSON | Description | Value |
| --- | --- | --- |
| { |  |  |
| " Enable": 0 | Every time the value changes, the device restarts. | 0: HTTPS not enabled 1: HTTPS Startup Status |
| " Port ": 443, | HTTP port |  |
| " RootCrtPath ":"/etc/ssl/ca.crt ", | The root Certificate path | Do not change it |
| "ServerCrt": { |  |  |
| "CommonName": "192.168.1.24", | Domain name or IP address of the Device | Like 192.168.1.24 |
| "Country":"China", | Conutry | Like China , USA and so on |
| "State": "zhejiang" | In China this means province | Like zhejiang |
| "Locality": "Hangzhou" | The detail position | Like hangzhou, default can be none |
| "Organization": "Wanjiaan" |  |  |
| "OrgUnit": "none" |  |  |
| "Emailaddress": "zb1208@126.com" | Email |  |
| "Path": "/mnt/mtd/Config/server.crt", | The path for install the certificate | Do not change |
| "UsefulLife": "365" | How long to use. |  |
| } |  |  |
| " Reserve ": "", |  |  |
| } |  |  |

    - /CGI/NetService/certmanager/exportrootcert

Get the certificate of the https from the Device. The download certificate need to be installed in the Local computer as a trusted certificate.

Example:

request URL: http://192.168.1.24/CGI/NetService/certmanager/exportrootcert

Response: -----BEGIN CERTIFICATE-----

MIIC7zCCAdegAwIBAgIBATANBgkqhkiG9w0BAQsFADAyMRAwDgYDVQQDEwdHZW5l

cmFsMREwDwYDVQQKEwhaaGVKaWFuZzELMAkGA1UEBhMCQ04wHhcNMDEwMTAxMDAw

MDAwWhcNMzAxMjMxMjM1OTU5WjAyMRAwDgYDVQQDEwdHZW5lcmFsMREwDwYDVQQK

EwhaaGVKaWFuZzELMAkGA1UEBhMCQ04wggEiMA0GCSqGSIb3DQEBAQUAA4IBDwAw

ggEKAoIBAQCTxj5Vjog/OL+Uk3ec3uvTAuXWxpTam6LRxkxw3qqtc6LG1WS9FQS3

bFACu8VJu+DX4y+PGfaWVUxwniWjTAevXS5tledVOp6D0/enjWCkK4HUHDgWCxu6

480sbqcUF4umVBKna5ZYKj4TEHib5DfkYxet8T3nnsftJljzP6vGL5t7RX4OLYMa

hqjQ+YLxeVsYJv/V4LB1PukEhOBd1uq+6idxzjt0XUJgj1MTr82OTuuElmsq638n

4VZjc/4ysKCLrhV+RRSQX+jWD7ZdNXBO/JbSyrKTZVEEO6pRhTRVF1sMbbaNeAEc

rvijhCEvjkbYKaLlBtJLqr7LJhyiLA9nAgMBAAGjEDAOMAwGA1UdEwQFMAMBAf8w

DQYJKoZIhvcNAQELBQADggEBACy+kYAtcxWGKKonPlU0zwxPukahwaSQ+8D/EluZ

sb6Fd69Y8qWFsnxrs2I/ICw+d+JZYA85Sq8g3UPz90/vDuh2W9N8HqVul+8+jCk2

YODx/OUNQ5DEFqRg7bXx1TtW0RrH4H1q7KuBeNdFjVP9uTV2DXgln3hjNkmYH0es

l7/443gX3RJaOsGIo6e0x57akwVOVL70bh52IuOIt7/cUPkwT6EcBjStrM5tLZYQ

EaEXOZAHG4fd9xsgDaeJw//BmndhJ1DyAYyHEZUEeUNYm8wexwOeMrUKppjkUygK

sA6K7nEazzU1g8z5yTcMCU7BgrefV3DHXFW2Dyym94bvsY4=

-----END CERTIFICATE-----

The certificate must saved as an file, and then to install in the local computer.

    - /CGI/NetService/certmanager/svrcertinfo

Get the install certificate infomation. Only support Get. Just for showing.

| JSON | Description | Value |
| --- | --- | --- |
| {{ "issuer": { "CommonName": "General", "Country": "CN", "Email": "", "Locality": "", "Organization": "ZheJiang", "OrganizationUnit": "", "State": "" }, "subject": { "CommonName": "192.168.1.24", "Country": "con", "Email": "zb1208@126.com", "Locality": "hangzhou", "Organization": "taoshi", "OrganizationUnit": "kejibu", "State": "zhejiang" }, "usefulLife": { "End": "2020-08-02 00:00:00", "Start": "2019-08-03 00:00:00" } } |  |  |

    - /CGI/NetService/certmanager/removesvrcert

Remove the certificate from the device.

    - /CGI/NetService/certmanager/importsvrcert

Import ssl certificate

Ps:The ssl certificate and the corresponding private key need to be imported.

Example:

{

certificateData: {

certificate:"-----BEGINCERTIFICATE-----\nMIID/jCCAuagAwIBAgIGAJmZmRAAMA0GCSqGSIb3DQEBCwUAMIGaMQswCQYDVQQG\nEwJVUzETMBEGA1UECAwKV2FzaGluZ3RvbjEQMA4GA1UEBwwHU2VhdHRsZTEUMBIG\nA1UECgwLRXhhbXBsZS5jb20xEjAQBgNVBAsMCUxpY2Vuc2luZzEUMBIGA1UEAwwL\nZXhhbXBsZS5jb20xJDAiBgkqhkiG9w0BCQEWFWxpY2Vuc2luZ0BleGFtcGxlLmNv\nbTAeFw0xNTEwMjkwMDU3MTVaFw0yNTEwMjYwMDU3MTVaMIGIMQswCQYDVQQGEwJV\nUzETMBEGA1UECAwKV2FzaGluZ3RvbjEUMBIGA1UECgwLRXhhbXBsZS5jb20xEzAR\nBgNVBAsMClByb2R1Y3Rpb24xEjAQBgNVBAMMCWxvY2FsaG9zdDElMCMGCSqGSIb3\nDQEJARYWcHJvZHVjdGlvbkBleGFtcGxlLmNvbTCCASIwDQYJKoZIhvcNAQEBBQAD\nggEPADCCAQoCggEBAMsnebbQ8YQD0KsajXKFR5/wWyhtUDU14Oefaa9U4dxmvG8n\ncN20eSjtStgfLYE3UdM0DtoUtpiuPoO/4o22uwsOE3yf6d80EZqgl7UNaAwsnIkq\noY3y0n32jiSVKix/gvIjdGX2mie8sT1Q+/oJRY0pwdpUoMgzFHuAIp6BZ+p2rTY8\ncsRMHqaquUJG93LANY7tfVLMTGGQLaS/knahp0TJLOb8OxS1ahFwLdN3vCCw+Lkl\nONn6KGlf7eh3hQ1KFJ/541ZZVOKhuww9PQ3Vqjw602QtWnQ4XsdbIs78Asi+Q1VV\nR0XPo9EauXQ5E5BD5qHAH2q6kV5/fPmx5tACy+sCAwEAAaNaMFgwCQYDVR0TBAIw\nADAdBgNVHQ4EFgQUVIKy9kYUEKgzlxrzy2UiKjFARgQwHwYDVR0jBBgwFoAUStYL\nMguPr/g+md/g4HCzp6YGeSgwCwYDVR0PBAQDAgXgMA0GCSqGSIb3DQEBCwUAA4IB\nAQAtjZFgLn48+9Cvi5+0etzsKYLpJWKSIl5pl/bWvKKsgv6Rg5x8J0WgzoYIuTfg\nhcCnX/mb5bbam6dOC9kM3YX5EHe7d74sScfvpRZUkG/Qbsg11TLIGyRl1s2RlV6Y\naDMUU+pPsi3ncc+7zKzNZuYyatxVF7J23DlrcUNZBNXK/18K4c78Ub/+40AyEddt\nnen3lhhx912rjYlaNVteKbB15Y5x3CtD8UJYD/GNX0Azrdc5iESsjE4SYOIpWf6w\nrceC3DeDW24UG+zBCLJ8AZiyDM4gloBG40bdpq8TUKuVoRBxcNUI3StlQJ/8hHc/\nJRX5EeYN923b81EhcG99e+i/\n-----END CERTIFICATE-----\n",

privateKey:"-----BEGINRSAPRIVATEKEY-----\nMIIEpAIBAAKCAQEAyyd5ttDxhAPQqxqNcoVHn/BbKG1QNTXg559pr1Th3Ga8bydw\n3bR5KO1K2B8tgTdR0zQO2hS2mK4+g7/ijba7Cw4TfJ/p3zQRmqCXtQ1oDCyciSqh\njfLSffaOJJUqLH+C8iN0ZfaaJ7yxPVD7+glFjSnB2lSgyDMUe4AinoFn6natNjxy\nxEwepqq5Qkb3csA1ju19UsxMYZAtpL+SdqGnRMks5vw7FLVqEXAt03e8ILD4uSU4\n2fooaV/t6HeFDUoUn/njVllU4qG7DD09DdWqPDrTZC1adDhex1sizvwCyL5DVVVH\nRc+j0Rq5dDkTkEPmocAfarqRXn98+bHm0ALL6wIDAQABAoIBAHxLVbOdmPa9YtEc\n+MfFdbIMW6mbYAR+R3FPXljwpRO1I7saNXrf7XU14aHjSXMShpSi+CdDnwhBK2xu\nXU15LG0yGogYSv3OHKsvcEJMxokUTl254Ig5tcFttT/6p4f8KAC/4npz65M+Zm5u\newxa2dvpXXqYZpq2beS12/iAkjSnej0urz6Q6aIb5xqPWspC+lf2AD1VDEBrLgjA\n1iE8kjI3qTqAAWQEkwfXSQ6LfDGSRzxyK2b9lTfH2g2mvBSkP28bLYVkaNSx/HRX\nB3rXnXenAFWQCbAcM0ttFYCEHdb2tQpJRX2bzcuhIJIZZMLlQTveB7RudrNKcAdD\nUfYWBwECgYEA7aAfcOTt8dMX3aSnsbx0FZcquBjgVU/LGbncyrJ6mg6sHV5oxjcQ\n/CLzLsQlRaSoVvnS5ldcLemMVeO0Y87eHfqoiVvZDO+MKzljv8YKPTH7Na0y4caf\nKkUjJFGklJzOI1VUbIaR9esDMIlWFrAp6IM4fcOHBoX7mmQ+Q+Qq72sCgYEA2tz7\nYJkXrQ2PkYh/zqMMrZVL7X9nEVHPZ7ISJDUKAxY+L+VFdSJAJB+5QQndxK6E+/tD\nYi1DOKTWTuzpOLrmCPKrLxv2eehg6TLLAqxJTRV066WfdwLExT3eu4zttpc4TUxX\nTWG+gohKhTNGSX6AVey8Otcdtn3so+Vov9ctNYECgYEAvEn23+Pmz4mvDYedAW8M\npNcFmJ7Nk4wIqTfnRg4qq8wYgaT/bKDRqpifDV2LEyYn5dNLVA3BEvhOpPgrRjLz\nOdoWldDbqi5eaRmuepUYI+lo3t2Axfo2wK3L7Mlsmf2uMb05ik0Pd0djXaBVQOES\nNBVWPAElCvw7o9jupak0hEUCgYEAjh4IrYU7toejvmDgfRGrXdwgXavpBXoWL8OV\nvCUkxzReVUOPgqwm8k4XQYBinSCmAen6q3mwuMtwzfdQyiGy1gSd3A1QU4DtzP/K\ni4XMY12Xep0lm2YU1++A/lBY28P+L1NvLTRXPPO7UBsnfPnD5AfBgoPFK4dBbnu9\nNTVIigECgYBSRYVro0dgTx55VuVwLxxaYX/5jMVy+V8rK+NAVlpFEGN7zzSuvdGT\n3FAsdbTI3XnKFdAwosptipQGT05f1hiKHVoQoVgLMMKFfFnVPDAgyaDiDdnmMSgB\nNVysXjI95zTPZKSVacK7bFu1xE2NM+KjcgK0lSXzSN9XiD8qTi5W/Q==\n-----END RSA PRIVATE KEY-----\n"

}

}

| JSON | Description | Value |
| --- | --- | --- |
| { certificateData: { |  |  |
| Certificate: | Contents of ssl certificate |  |
| privateKey: | Private key content |  |
| }} |  |  |

    - /CGI/NetService/certmanager/exporsvrtcert

Export the certificate from the device.

Example:

{

certificateData:"-----BEGINCERTIFICATE-----\nMIIDxzCCAq+gAwIBAgIBATANBgkqhkiG9w0BAQsFADB5MQswCQYDVQQGEwJDTjEN\nMAsGA1UECAwETm9uZTENMAsGA1UEBwwETm9uZTEQMA4GA1UECgwHR2VuZXJhbDEN\nMAsGA1UECwwETm9uZTEWMBQGA1UEAwwNMTkyLjE2OC4zLjExMzETMBEGCSqGSIb3\nDQEJARYETm9uZTAeFw0yNTA3MDMwNTU2MzdaFw0yNjA3MDMwNTU2MzdaMHkxCzAJ\nBgNVBAYTAkNOMQ0wCwYDVQQIDAROb25lMQ0wCwYDVQQHDAROb25lMRAwDgYDVQQK\nDAdHZW5lcmFsMQ0wCwYDVQQLDAROb25lMRYwFAYDVQQDDA0xOTIuMTY4LjMuMTEz\nMRMwEQYJKoZIhvcNAQkBFgROb25lMIIBIjANBgkqhkiG9w0BAQEFAAOCAQ8AMIIB\nCgKCAQEAz08MDMOSfivoaSiqnC4WYJXVfQ7HwLBIYX+xWHC3S8l2P6rybnPEH/D7\n97pN3JRoOYD83AL3VbDBg7gXsOWBsmVtNudi+gJM9I/eKhzr1/hSDwWwyMMQu2XP\nVqURXd3JaUUHen1FYXXTX6/0NKKnhIK4nRB63pTktSTF7HXoGKqp5MMJ1i5y1BSD\nwHflz5A3xdgJVJ7xcHF0iQ9B6yqgeYeFiksXpgw7rhlfqDhvetgLWScJ146S+IeQ\nxBpiwWtOdqQaQlSuUFThYmgS1J7Vcj4L3V/bR/cCRi8itPQo7JxNEO1BrYYPQPHE\ndYNXfLP0P3KuOPxAtsQm4pD8MJ8GAQIDAQABo1owWDAJBgNVHRMEAjAAMAsGA1Ud\nDwQEAwIFoDAdBgNVHQ4EFgQUpF92qH8Ysv9I+2JtMs2cpEv6H1EwHwYDVR0jBBgw\nFoAUpF92qH8Ysv9I+2JtMs2cpEv6H1EwDQYJKoZIhvcNAQELBQADggEBAD5Z2LEf\neH9K0v7slr4NVSzGZ7A81TYMeZEwYeVj32ByskwMwONfWItXmb/qEtkXFbiicEAb\n09SzPAAwylTPO5g7C0lXUFvm1gyHN2b6phbFfvRb3vGFEZ0ED5YqE+deibqbWMlg\n8h88kgWnHIcPxaWPjhkph9bErtToItkvhwX9560kWRl4nBzMx9k0JUOL2FyBr5D7\nQiHjeIRI7aUc8WPfuNGMQo99jaCeQZy10zk8vseEPHQEg+dd6MMl+IY0biOB/oXW\nORBqDMccs9D5kpenbpyDWqVcPR/rljeNlYnZUZV58gjeqMZDaBEKPPuLkuuMLqfu\nAQXZfnVOd62Gis4=\n-----END CERTIFICATE-----\n",

statusCode: 1,

statusString: "OK"

}

The certificate must be saved as a file before it can be downloaded.

| JSON | Description | Value |
| --- | --- | --- |
| { certificateData: |  |  |
| statusCode |  |  |
| statusString: |  |  |
| }} |  |  |

    - /CGI/NetService/otaupgrade

GET/PUT OTA upgrade configuration

| JSON | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Check": false, |  | true, false |
| "FileName": "", |  |  |
| "Version":"", | Version information, can not be changed. |  |
| "BuildDate":"", | Version Generation Date. |  |
| "RelNotes":"", | Update Contents. |  |
| } |  |  |

    - /CGI/NetService/upgradestate

GET Used to click on the "Upgrade" operation to get the status of the upgrade

| JSON | Description | Value |
| --- | --- | --- |
| { |  |  |
| "State":"NoUpgrade", | When the value is "NoUpgrade", it is not downloaded. When the value is "Download Process", download progress. When the value is Download Error, the download fails. When the value is "Upgrade Process", it is in progress for the upgrade. When the value is "Upgrade Finish", the upgrade ends. When the value is "Upgrade Error", the upgrade fails. | DownloadProcess, DownloadError, UpgradeProcess, UpgradeFinish, UpgradeError, NoUpgrade |
| "Process":-1, | When the "State" field is "Download Process" and "Upgrade Process", the value of "Process" represents progress 0 - 100. When the "State" field is "Download Error" and "Upgrade Error" "Process" is -1 | -1,0-100 |
| } |  |  |

    - /CGI/NetService/httpupgrade

GET/PUT Get OTA Upgrade Configuration

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| "AutoCheck":6, |  | Int |
| "AutoUpgrade":1, |  | Int |
| "Enable": 0, | When the value is 1, the automatic upgrade key is updated to the selected state. When the value is 0, the automatic upgrade key is updated to an unchecked state. | Int 0,1 |
| "IgnoreDate":"", |  |  |
| "Port":80, |  |  |
| "ServerAddr": "ota.instaon.com", |  |  |
| } |  |  |

    - /CGI/Upgrade/init

POST  Initialize the web upgrade (non-OTA), Trigger the local upgrade through this interface.

Transmission preparation

Request parameters:

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| "filename": "firmware_v1.2.3.bin", | Firmware file name | string |
| "file_size": 1048576, | File size | Int |
| "status": "start" | Ask if the device can start transmitting the firmware |  |
| } |  |  |

Response parameters:

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| status": "ok", | ok or error | string |
| "message": "" | Return when an error occurs. | string |
| } |  |  |

Firmware data transmission

Response parameters:

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| status": "ack", | Return each time the block data is received | string |
| "bytes_received": 65536 | The size of the data received each time | int |
| "total_received": 131072 | The total size of the received data | int |
| "message":"" | Return when an error occurs. |  |
| } |  |  |

Transmission completed

Request parameters:

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| "status": "complete", | Tell the device firmware that the transmission has been completed | string |
| } |  |  |

Response parameters:

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| status": "ok", | ok or error | string |
| total_received: 1048576 | The total received data |  |
| "message": "" | Return when an error occurs. | string |
| } |  |  |

    - /CGI/NetService/librtmp

For IPC: /CGI/NetService/librtmp

GET/PUT the librtmp configuration

Network Config->Net Service->RTMP

JSON：

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| "maxSrvSize":1, | max server size | int readonly |
| "MediaStream":[{ |  |  |
| "rtmpEnable":false, | Enable switch | false: close the function true: open the function |
| "rtmpsEnable":false, | Enable switch | false: close the function true: open the function |
| "rtmpServerUrl":"", | RTMP server URL link | rtmp://ServerIP:1935/live/stream |
| "rtmpsServerUrl":"" | RTMPS server URL link | rtmps://ServerIP:19350/live/stream |
| "rtmpMainStream": 0, | 0:Main Stream 1: Sub Stream |  |
| "rtmpsMainStream": 0 | 0:Main Stream 1: Sub Stream |  |
| }] |  |  |
| } |  |  |

    - /CGI/NetService/debug

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| "sshDebug":true, | Enable switch | false: close the function true: open the function |
| } |  |  |

    - /CGI/NetService/webSocket

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| "bEnable":true, | Open or close function | false: close the function true: open the function |
| } |  |  |

    - /CGI/NetSerice/snmp

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| "bEnable":true, | Open or close function | false: close the function true: open the function |
| "snmpType": 0, |  | 0:SNMPV3 1:SNMPV2 |
| "UserName": "admin", |  |  |
| "TypeList": [ "SNMPV3", "SNMPV2", ], |  |  |
| "AuthenMode": 0, | Authentication Mode | 0: MD5 |
| "AuthenList": [ "MD5", ], |  |  |
| "AuthenPwd": "", |  |  |
| "EncryptMode": 0, | Encryption Mode | 0: DES |
| "EncryptList": [ "DES", ], |  |  |
| "EncryptPwd": "", |  |  |
| "TrapCmtyName": "private", | Trap Community Name | read only |
| "TrapSvrAddress": "0.0.0.0", | Trap Server Address | read only |
| "TrapPort": 162, |  | read only |
| "SNMPPort": 161, |  |  |
| } |  |  |

- 2.5.27. /CGI/NetPlatform/net4g

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Apn":" ", | Current device APN | Set by the user, the device saves it and takes effect, and it is displayed on the front end. |
| "iccid":" " | The ICCID of the SIM card | Read-only,No editing allowed. |
| } |  |  |

  - Network

Note: For configure API, every one i will give out the body, If for “GET” operation do not need to send the body. And for “PUT” operation  need to send the body. If some field write “no care”, “do not change” and so on, please send back the value as get.

Web page: Network Config->Net Service->TCP/IP will use these command

| Command | Description | GET | PUT |
| --- | --- | --- | --- |
| netcommon | General network configuration | √ | √ |
| dhcp | DHCP | √ | √ |
| mtu | MTU configuration | √ | √ |
| dns | DNS configuration | √ | √ |

    - /CGI/Network/netcommon

GET/PUT general network configuration

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| "GateWay": "0x0101A8C0", | Gate Way | 1.1.168.192 |
| "HostIP": "0x2D01A8C0", |  | 45.1.168.192 |
| "HostName": "LocalHost", | Host Name |  |
| "HttpPort": 80, |  |  |
| "MAC": "ce:22:12:13:cf:ae", |  |  |
| "MaxBps": 0, | No use Now |  |
| "MonMode": "TCP", | No use Now |  |
| "SSLPort": 8443, | For HTTPS |  |
| "Submask": "0x00FFFFFF", |  |  |
| "TCPMaxConn": 10, | Please do not change,is a setting by device |  |
| "TCPPort": 34567, |  |  |
| "TransferPlan": "Quality", | no use |  |
| "UDPPort": 34568, | no use |  |
| "UseHSDownLoad": false | no use |  |
| } |  |  |

    - /CGI/Network/dhcp

GET/PUT network priority.

For some device maybe has more than one network card ,so the structure normally return 4, but normally only use one

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Enable": false, |  |  |
| "Interface": "eth0" |  |  |
| }, |  |  |
| { |  |  |
| "Enable": false, |  |  |
| "Interface": "eth1" |  |  |
| }, |  |  |
| { |  |  |
| "Enable": false, |  |  |
| "Interface": "eth2" |  |  |
| }, |  |  |
| { |  |  |
| "Enable": false, |  |  |
| "Interface": "eth3" |  |  |
| } |  |  |

    - /CGI/Network/mtu

GET/PUT MTU configuration

{ "MTU": 1500 }

    - /CGI/Network/dns

GET/PUT DNS Configuration

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Address": "0x08080808", | First DNS address | 8.8.8.8 |
| "SpareAddress": "0x04040808" | Second DNS address | 4.4.8.8 |
| } |  |  |

Web page: Network Config->Net Service->TCP/IP

    - /CGI/Network/ipv6

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| "GateWay": "fe80::1", | Gate Way |  |
| "HostIP": "", | Host IP |  |
| "LinkAddr": "fe80::4457:a9ff:fe6c:59ad", |  | ‌‌ |
| "IPNum": 64, |  |  |
| "NetDHCP": true, |  |  |
| } |  |  |

  - Alarm

Note: For configure API, every one i will give out the body, If for “GET” operation do not need to send the body. And for “PUT” operation  need to send the body. If some field write “no care”, “do not change” and so on, please send back the value as get.

| Command | Description | GET | PUT |
| --- | --- | --- | --- |
| motion/channels/<ID> | Motion detection configuration for single channel. IPC can use this | √ | √ |
| blind/channels/<ID> | Blind configuration for single channel. IPC can use this | √ | √ |
| alarmin/channels/<ID> | .Alarm In configuration for single channel,IPC can use this | √ | √ |
| alarmout | Alarm out configuration | √ | √ |
| abnormal | The abnormal alarm like disk error | √ | √ |

    - /CGI/Alarm/motion/channels/<ID>

For IPC: /CGI/Alarm/motion/channels/0

GET/PUT the motion detection configuration

Web page: Alarm->Alarm Config-> Motion Detection

JSON：

| Sample | Description | Value |
| --- | --- | --- |
| "Enable": false, | Open or close function | false: close the function true: open the function |
| "EventHandler": { | What will do when alarm happen. |  |
| } |  |  |
| "Level": 3, | Level of sensitivity | 1-6, 6 is the most sensitive |
| "Region": [ | The region that select to detect alarm. The video can mostly divided to 32*32 regions |  |
| "0x000000FE", | The first row, has 1*32 region | Use bit to express, so FE means 1111 1110 if 1 means the region is selected |
| "0x000000FE", |  |  |
| "0x000000FE", |  |  |
| ..... |  |  |
| ] |  |  |

    - /CGI/Alarm/blind/channels/<ID>

GET/PUT  Video occlusion alarm config

Web page: Alarm->Alarm Config-> Block Alarm

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Enable": true, | Open or close function | false: close the function true: open the function |
| "EventHandler": { | What will do when alarm happen.See the |  |
| } |  |  |
| "Level": 3, |  | 1-6, 6 is the most sensitive |
| } |  |  |

    - /CGI/Alarm/alarmin/channels/<ID>

GET/PUT  Sensor alarm config

Web page: Alarm->Alarm Config-> Alarm input

| Sample | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Enable": true, |  |  |
| "EventHandler": { | What will do when alarm happen.See the |  |
| } |  |  |
| "SensorType": "NO" |  |  |
| } |  |  |

    - /CGI/Alarm/alarmout

URL: http://192.168.1.41/CGI/Alarm/alarmout
GET/PUT  Sensor alarmout config

Web page: Alarm->Alarm Config-> Alarm Output

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "AlmOutType":0 |  | 0:Configure 1:Manual 2:Stop |
| "AlmOutState":0 | The alarm out state | 0: close 1: open |
| } |  |  |

    - /CGI/Alarm/abnormal/nodisk

GET/PUT the abnormal the configuration of nodisk

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Enable": true, |  |  |
| "EventHandler": { | See the EventHandle, normally this type we only use sound and Email |  |
| } } |  |  |

    - /CGI/Alarm/abnormal/diskerr

GET/PUT the abnormal alarm configuration of disk error

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Enable": true, |  |  |
| "EventHandler": { | See the Event Handler, normally this type we only use sound and Email |  |
| } } |  |  |

    - /CGI/Alarm/abnormal/disknospace

GET/PUT the abnormal alarm configuration of disk no space

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| Enable: true, |  |  |
| EventHandler: { | See the “EventHandler”, normally this type we only use sound and Email |  |
| } |  |  |
| LowerLimit: 5, | When the function is open,this means the space is lower than the setting value, will trigger alarm | percent |
| RecordTime: 0, | No use |  |
| RecordTimeEnable: false | No use |  |
| } |  |  |

    - /CGI/Alarm/abnormal/netdisconn

GET/PUT the abnormal alarm configuration of net work disconnect

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| Enable: true, |  |  |
| EventHandler: { | See the EventHandler, normally this type we only use sound and Email |  |
| } |  |  |
| } |  |  |

    - /CGI/Alarm/abnormal/ipconflict

GET/PUT the abnormal alarm configuration of ipconflict

| Json | Description | Value |
| --- | --- | --- |
| { |  |  |
| Enable: true, |  |  |
| EventHandler: { | See the EventHandler, normally this type we only use sound and Email |  |
| } |  |  |
| } |  |  |

    - /CGI/Alarm/region/channels/<ID>

For IPC: /CGI/Alarm/region/channels/0

GET/PUT the region intrusion detection configuration

Web page: Alarm->Alarm Config-> Region Intrusion

JSON:

| Sample | Description | Value |
| --- | --- | --- |
| { | Open or close function | false: close the function true: open the function |
| "Enable": false, |  |  |
| "bDrawBox": false, |  |  |
| "EventSupport":2, | Supported detection events | /*Pedestrian intrusion event mask */ EventSupport= 2, /*Vehicle intrusion event mask */ EventSupport= 4, /*Pedestrian or vehicle intrusion (either) event mask */ EventSupport= 6, |
| "RegionMax": 4, | The equipment can support a maximum of 4 detection areas. |  |
| "RegionConfig": [ |  |  |
| { | Region[0] |  |
| "EventMask": 2, | Turn open or close the detection event | /*Pedestrian intrusion event mask */ EventMask= 2, /*Vehicle intrusion event mask */ EventMask= 4, /*Pedestrian or vehicle intrusion (either) event mask */ EventMask = 6, |
| "Sensitivity": 50, | sensitivity | value range: 0-100 |
| "TimeCooldown":3, | Upload time for test results cooling down | Unit: Seconds |
| "InvasionDuration":3, | Duration of continuous invasion | Unit: Seconds |
| "Region":[ 20,10, 200,10, 200,120, 20,120 ], | The region that select to detect alarm. It is a rectangular area formed by four x1,y1,x2,y2,x3,y3,x4,y4 coordinate points. |  |
| }, |  |  |
| ... | Region[1] |  |
| ... | Region[2] |  |
| ... | Region[3] |  |
| ], |  |  |
| "EventHandler": {} | What will do when alarm happen. |  |
| } |  |  |

    - /CGI/Alarm/crossline/channels/<ID>

GET/PUT the cross line detection configuration

JSON:

| Sample | Description | Value |
| --- | --- | --- |
| { | Open or close function | false: close the function true: open the function |
| "Enable": false, |  |  |
| "TriggerMax": 4, | This device can support up to 4 detection rules at most. |  |
| "TriggerConfig":[ |  |  |
| { | TriggerConfig[0] |  |
| "Sensitivity": 50, | sensitivity | value range: 0-100 |
| "TriggerMethod": 0, | Alarm trigger method | 0x00: From left to right or from right to left 0x01: From left to right 0x02: From right to left |
| "TriggerLine": [ 110,10, 110,120 ], | The crossline that select to detect alarm. It is a line formed by two coordinate points, x1, y1 and x2, y2. |  |
| }, |  |  |
| ... | TriggerConfig[1] |  |
| ... | TriggerConfig[2] |  |
| ... | TriggerConfig[3] |  |
| ], |  |  |
| "EventHandler": {} | What will do when alarm happen. |  |
| } |  |  |

    - /CGI/Alarm/onekeydisarm

GET/PUT the One-Key Disarming configuration

JSON:

| Sample | Description | Value |
| --- | --- | --- |
| { | Open or close function | false: close the function true: open the function |
| "Enable": false, |  |  |
| "iTimerMode": 0, | Select a date or period to cancel the alarm | 0: Select TimeSection, 1: Select DateSchedule |
| "bMail": true, | Open or close function | bMail true: Open sending emails |
| "bVoice": true, | Open or close function | bVoice true: Turn on the sound |
| "bLight": true, | Open or close function | bLight true: Turn on the flashing lights |
| "DateSchedule": [ "2025-08-06 00:00:00", "2025-08-06 24:00:00" ], | Cancel the alarm by date and time |  |
| "TimeSection": [ "0 00:00:00-24:00:00", "0 00:00:00-24:00:00", "0 00:00:00-24:00:00", "0 00:00:00-24:00:00", "0 00:00:00-24:00:00", "0 00:00:00-24:00:00" ], | TimeSection[0] For details, please see Normal JSON structure – TimeSection | Array[6] String |
| [. . . . . . ], | TimeSection[1] |  |
| . . . . . . |  |  |
| [. . . . . . ], | TimeSection[6] |  |
| } |  |  |

  - UserManager

Web Page: System Config -> System Tool

| Command | Description | GET | PUT | POST | DELETE |
| --- | --- | --- | --- | --- | --- |
| grouplist | All the group or device | √ |  |  |  |
| userlist | All the user of device | √ |  |  |  |
| users/”username” | Add user |  |  | √ |  |
| users/”username”&”password” | Modify user’s authority or password |  | √ |  |  |
| users/”username” | Delete user |  |  |  | √ |
| groups/”groupname” | Add group |  |  | √ |  |
| groups/””groupname” | Modify group |  |  |  |  |
| groups/””groupname” | Delete group |  |  |  | √ |

    - /CGI/UserManager/grouplist

Get the group list.

| Json | Description |
| --- | --- |
| [ { "AuthorityList" : [ ...... ], | The Group authority |
| "Memo" : "administrator group", | Describe, this means administrator. This group has the most authority,and can not be change. |
| "Name" : "admin" | Group name |
| }, |  |
| { "AuthorityList" : [ ...... ], |  |
| "Memo" : "user group", "Name" : "user" } | The group name is user, and is user group. The group has only the needed authority, the administrator can allocate |
| ] |  |

    - /CGI/UserManager/userlist

Get the user list and the list

| Json | Description |
| --- | --- |
| [ { "AuthorityList" : [ ...... ], | Authority that the user has |
| "Group" : "admin", | User belongs to the “admin” group |
| "Memo" : "admin 's account", | description |
| "ModifyTime" : "", | Use when modify |
| "Name" : "admin", | The user name |
| "Password" : "Ny2SRZzv", | The password of the user |
| "Reserved" : true, | No use |
| "Sharable" : true | This must be true. |
| } |  |
| { "AuthorityList" : [ ], "Group" : "user", "Memo" : "guest 's account", "ModifyTime" : "", "Name" : "guest", "Password" : "tlJwpbo6", "Plain" : "", "Reserved" : true, "Sharable" : true } | The guest user’s information |
| ] |  |

    - /CGI/UserManager/users/“username”

Add one user, should set the user’s authority and the belongs group.

For example :

URL: http://192.168.1.238/CGI/UserManager/users/ccc

| Json | Description |
| --- | --- |
| [ "Group":"admin", | The group that the user belongs to. |
| "Name":"ccc", | The user name |
| "Password":"tlJwpbo6", | The user’s password，Encrypted. |
| "Memo":"sss", | Just description |
| "ModifyTime":"", | Use when modify |
| "Reserved" : true, | No use now |
| "Sharable" : true | This must be true. |
| "AuthorityList":[ ...... | The added user’s authority |
| ] |  |
| } |  |
| ] |  |

    - /CGI/UserManager/users/”encrypted(“username”&“password”)”

Modify the user’s authority or password.

For example:

The really URL: http://192.168.1.174/CGI/UserManager/users/ZPUHZMM50I76u6m%2BwSy3DA%3D%3D%2624

The username is “zbtest” and password is “admin123”. through the later step：

1：Step1 = BASE64(DES(username + "&" + md5(password)）

2：Step2 = URLencode(Step1 + "&" + Step1 .length);

Step2 = ZPUHZMM50I76u6m%2BwSy3DA%3D%3D%2624

More Encryption Algorithm if needed can show the code.

Method:PUT

| Json | Description |
| --- | --- |
| [ "Group":"admin", | The group that the user belongs to. |
| "Name":"ccc", | The user name |
| "Password":"tlJwpbo6", | The user’s password ,Encrypted. |
| "Memo":"sss", | Just description |
| "ModifyTime":"", | Use when modify |
| "Reserved" : true, | No use now |
| "Sharable" : true | This must be true. |
| "AuthorityList":[ ...... | The user’s authority, |
| ] |  |
| } |  |
| ] |  |

    - /CGI/UserManager/users/“username”

Delete user

For example:

URL:  http://192.168.1.238/CGI/UserManager/users/ccc

Method: DELETE

    - /CGI/UserManager/groups/”groupname”

Add new group

URL: 	http://192.168.1.238/CGI/UserManager/groups/2dsURL

Method： POST

| Json | Description |
| --- | --- |
| [ "Name":"dddd"," | The new group name |
| "Memo":"", | The group’s description |
| "AuthorityList":[ ...... | The group’s authority, |
| ] |  |
| } |  |
| ] |  |

    - /CGI/UserManager/groups/”groupname”

Modify  group

URL:		http://192.168.1.238/CGI/UserManager/groups/2dsURL

Method： PUT

| Json | Description |
| --- | --- |
| [ "Name":"dddd"," | The new group name |
| "Memo":"", | The group’s description |
| "AuthorityList":[ ...... | The group’s authority, |
| ] |  |
| } |  |
| ] |  |

    - /CGI/UserManager/groups/”groupname”

Delete  the group

For example:

URL: http://192.168.1.238/CGI/UserManager/groups/dddd

Method: DELETE

  - CameraParam

Web page: Camera -> Output Model;

    - /CGI/Preview/prevcover/channels/<ch.no>

Url: http://192.168.1.41/CGI/Preview/prevcover/channels/0  (For example, address depends on yourself)

Type: GET/PUT

Note: Both the osd info and the privacy mask are in this json structure.

| JSON | Description | Value |
| --- | --- | --- |
| "ChannelTitle": { "Name": "HD-IPC" }, | Modify the OSD channel title. key: ChannelTitle.Name value : max size should be less then 64bytes | String. Support the special symbol like ‘@#!$..’ |
| "ChannelTitleAttribute": { | The attribute of channel title↓ |  |
| "BackColor": "0x00000000", | The color of channel title’s background |  |
| "EncodeBlend": true, | Enable(show or not show) the channel title | Bool |
| "FrontColor": "0x00FFFFFF", | The color of channel title’s text |  |
| "PreviewBlend": true, | Not in use now | Bool |
| "RelativePos": [0, 8168, 255, 8192] | The relative position where is the channel title box located. | Array |
| }, | The attribute of channel title↑ |  |
| "TimeTitleAttribute": { | The attribute of time title↓ |  |
| "BackColor": "0x00000000", | The color of time title’s background |  |
| "EncodeBlend": true, | Enable(show or not show) the time title | Bool |
| "FrontColor": "0x00FFFFFF", | The color of time title’s text |  |
| "PreviewBlend": true, | Not in use now | Bool |
| "RelativePos": [7937, 0, 8192, 24] | The relative position where is the time title box located. | Array |
| }, | The attribute of time title↑ |  |
| "Covers": [ | The info of privacy mask, length=8 ↓ | Array |
| { | Covers[0] |  |
| "BackColor": "0x80000000", | The color of cover block’s background |  |
| "EncodeBlend": false, | Enable this cover block(Covers[0]) | Bool |
| "FrontColor": "0x00FFFFFF", | Not in use now |  |
| "PreviewBlend": false, | Not in use now | Bool |
| "RelativePos": [1024, 1024, 2048, 2048 ] | The relative position where is the covers[0] block located. | Array |
| }, |  |  |
| {. . . . . .} | Covers[1] |  |
| {. . . . . .} | Covers[2] |  |
| {. . . . . .} | Covers[3] |  |
| {. . . . . .} | Covers[4] |  |
| {. . . . . .} | Covers[5] |  |
| {. . . . . .} | Covers[6] |  |
| {. . . . . .} | Covers[7] |  |
| ], | The info of privacy mask↑ |  |
| "CoversNum": 0, | The num of cover blocks you enabled | Int |
| "CustomTitle": { "strTitle": "" }, | Modify the OSD custom title. key: Custom Title String value : max size should be less then 64bytes | String. Support the special symbol like ‘@#!$..’ |
| "CustomTitleAttribute": { | The attribute of channel title↓ |  |
| "BackColor": "0x00000000", | The color of channel title’s background |  |
| "EncodeBlend": true, | Enable(show or not show) the channel title | Bool |
| "FrontColor": "0x00FFFFFF", | The color of channel title’s text |  |
| "PreviewBlend": true, | Not in use now | Bool |
| "RelativePos": [ 7937, 8168, 8192, 8192] | The relative position where is the channel title box located. | Array |
| "SignalTitleAttribute": { | The attribute of channel title↓ |  |
| "BackColor": "0x00000000", | The color of channel title’s background |  |
| "EncodeBlend": true, | Enable(show or not show) the channel title | Bool |
| "FrontColor": "0x00FFFFFF", | The color of channel title’s text |  |
| "PreviewBlend": true, | Not in use now | Bool |
| "RelativePos": [ 7937, 8168, 8192, 8192] | The relative position where is the channel title box located. | Array |

    - /CGI/Preview/colorset/channels/<ch.no>

Url:  http://192.168.1.41/CGI/Preview/colorset/channels/0  (For example, address depends on yourself)

Type: GET/PUT

| JSON | Description | Value |
| --- | --- | --- |
| [ |  |  |
| { | colorset/channels/0 “json”[0] ↓ |  |
| "Enable": true, | Enable colorset/channels/0 “json”[0] | Bool |
| "TimeSection": "0 00:00:00-24:00:00", | Not in use now. Please let it be ”0-24” all the way. | String |
| "VideoColorParam": { | The parameters of video color set ↓ |  |
| "3DNr": 0, | 3D Noise filter, not in use now | Int |
| "Acutance": 2056, | Acutance, Horizontal acutance = Acutance &0x00000000f. Vertical acutance = (Acutance &0x000000f00)>>8 | Int |
| "Brightness": 50, | Brightness | Int 0-100 |
| "Contrast": 50, | Contrast | Int 0-100 |
| "Gain": 0, | Gain | Int 0-100 |
| "Hue": 50, | Hue | Int 0-100 |
| "Saturation": 50, | Saturation | Int 0-100 |
| "SfStrength": 0, | Not in use now | Int |
| "TfStrength": 0, | Not in use now | Int |
| "Whitebalance": 128 | White balance | Int |
| } | The parameters of video color set ↑ |  |
| }, | colorset/channels/0 “json”[0] ↑ |  |
| {. . . . . .} | colorset/channels/0 “json”[1] Ibid. |  |
| ] |  |  |

    - /CGI/Custom/cameraparam

Url: http://192.168.1.41/CGI/Custom/cameraparam (For example, address depends on yourself)

Type:  GET/PUT

| JSON | Description | Value |
| --- | --- | --- |
| [ | All channel’s camera parameters, 32 channels |  |
| { | The camera parameters of channel[0]↓ |  |
| "ApertureAuto": 0, | Automatic aperture, 0-close; 1-open | int |
| "DNCThreshold": 10, | Day and night switching threshold | int 0-100 |
| "DayNightMode": 0, | Day & Night mode, 0-auto/infrared starlight; 1-colorful/day; 2-wite or black/night | int |
| "DNSensitivity":0 | The sensitivity for switching between day and night | int 0-Medium 1-High |
| "ElecShutter": 0, | Electronic shutter, 0-none, 1-weak; 2-medium; 3-strong | int |
| "FocusMode": 0, | Focus mode, 0-auto; 1-manual; 2-half-auto | int |
| "IRCutSwap": 0, | IRCUT reverse order, 0-normal; 1-reverse |  |
| "IREndTime":"2000-01-02 07:00:00", | The end of the day. |  |
| "IRStartTime":"2000-01-01 18:00:00", | Daytime start time. |  |
| "IRSwitchMode": 0, | IRCUT switching mode, 0- infrared light synchronous switching; 1-auto switching | int |
| "ImageFlip": 0, | Image flip, 0-normal; 1-flip | int |
| "ImageMirror": 0, | Image mirror, 0-normal; 1-mirror | int |
| "WDRMode": 0 | WDR mode, 0-close; 1-open 2-auto | int |
| "WDRLevel": 50 | 1-100 Settings when WDRMode is open | int |
| "Reserve": "", |  |  |
| "Reserve1": "", |  |  |
| "WDRMode": 0 |  | int |
| }, | The camera parameters of channel[0]↑ |  |
| {. . . . . . } | The camera parameters of channel[1] |  |
| . . . . . . |  |  |
| {. . . . . . } | The camera parameters of channel[31] |  |
| ] |  |  |

    - /CGI/Custom/cameraispparam

Url:http://192.168.1.41/CGI/Custom/cameraispparam (For example, address depends on yourself)

Type:  GET/PUT

| JSON | Description | Value |
| --- | --- | --- |
| [ | All channel’s ISP parameters, 32 channels |  |
| { | The ISP parameters of channel[0]↓ |  |
| "DNState": 0, | The state of day&night, when the DayNightMode(this parameter in cameraparam) is auto, current used total parameter is TotalParam[DNState]. | Int 0-3 |
| "TotalParam": [ | Total parameters Depends on cameraparam and DNState: If(cameraparam. DayNightMode===1 && cameraparam. WDRMode===0 ){ TotalParam[0], }else if(cameraparam. DayNightMode===2 && cameraparam. WDRMode===0){ TotalParam[1], } else if(cameraparam. DayNightMode===1 && cameraparam. WDRMode===1){ TotalParam[2], } else if(cameraparam. DayNightMode===2 && cameraparam. WDRMode===1){ TotalParam[3], }else{ TotalParam[DNState], } ↓ |  |
| { | TotalParam[0] ↓ |  |
| "AESensitive": 5, | AE Sensitive | Int 1-10 |
| "AntiFlicker": 0, | Fluorescent lamp, 0-close, 1-open | Int |
| "AntiFlickerlevel":0 | Line Flicker Suppression 0-100 def:0 | int |
| "Bias":0 | Exposure Compensation,0 ~ 100 def:0 | int |
| "Metering": | Metering Control, 0-3, def:0 | int |
| "ShutterSpeed":12 | Shutter speed | int 12 - 10000 |
| "SlowShutterParm": { "Enable": 0, "Speed":1 } | Slow shutter speed | Speed:1-25 Enable: 0-off 1-on |
| "BLCEnable": 0, | Backlight compensation enable, 0-close, 1-open | Int 0,1 |
| "BLCLevel": 0, | Backlight compensation level | Int 0-100 |
| "DefogEnable": 0, | Enable defogging, 0-close, 1-open | Int 0,1 |
| "ExpRefLevel": 60, | Automatic exposure reference | Int 1-255 |
| "ExposureParam": { "Level": 0, "LeastTime": "0x100", "MostTime": "0x10000" }, | Config exposure parameters: Level: exposure level, 0-auto; LeastTime: Not in use now(Unit: ms), MostTime:Not in use now(Unit: ms) | Level:int 0-8 |
| "GainParam": { "AutoGain": 1, "Gain": 50 }, | Config gain parameters: AutoGain: enable auto gain, 0-close, 1-open, Gain: gain | AutoGain:int 0,1 Gain:int 0-100 |
| "NFLevel": 0, | Noise reduction level, 0-auto | Int 0-5 |
| "Reserve": "", |  |  |
| "UserModify": 0, | Whether the user has modified, 0-no, 1-modified | Int 0,1 |
| "WBMode": 0, | Scene mode, 0-auto, 1-low light priority; 2-high light priority | Int |
| "WDRLevel": 50, | Set wide dynamic range. Value: 0 - 100, default: 50. | It is effective when WDRMode is set to "manual". |
| "LampSourceMode": "Soft Light", | Light source mode |  |
| "LampSourceMax": 2, |  |  |
| "LampSourceList": [ |  |  |
| "Soft Light", |  |  |
| "Warm Light" |  |  |
| "Infrared Hard Light" |  |  |
| "Smart White Light |  |  |
| "Full color" |  |  |
| ] |  |  |
| }, | TotalParam[0] ↑ |  |
| {. . . . . . }, | TotalParam[1] |  |
| {. . . . . . }, | TotalParam[2] |  |
| {. . . . . . }, | TotalParam[3] |  |
| }, | The ISP parameters of channel[0]↑ |  |
| {. . . . . .}, | The ISP parameters of channel[1] |  |
| . . . . . . |  |  |
| {. . . . . .}, | The ISP parameters of channel[31] |  |
| ] | ISP parameters over |  |

    - /CGI/Custom/cameraprofile

Url:http://192.168.1.41/CGI/Custom/cameraprofile

Type:  GET/PUT

| JSON | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Always": 0, | When the value is 0, corresponding to the day, 1 is corresponding to the night. | Int 0,1 |
| "DayTime": "1 00:00:00-24:00:00", | Day in set time, otherwise night. | String |
| "Mode":1, | When the value is 1, the profile management corresponds to full-time and is 2. The profile management corresponds to time switching. | Int 1,2 |
| "Reserve": "", |  |  |
| } |  |  |

    - /CGI/Preview/texttitle/channels/<ch.no>

Url: http://192.168.1.41/CGI/Preview/texttitle/channels/0 (For example, address depends on yourself)

Type: GET/PUT

| JSON | Description | Value |
| --- | --- | --- |
| "CustomTitle": { | Config title text parameters ↓ |  |
| "BackColor": "0x80000000", | The color of overlay text box background |  |
| "EncodeBlend": false, | Enable(show or not show) the overlay text | Bool |
| "FrontColor": "0x00FFFFFF", | The color of overlay text |  |
| "PreviewBlend": false, | Not in use now |  |
| "RelativePos": [ 7040, 7680, 8192, 8192] | The relative position where is the overlay title box located. | Array |
| }, | Title text parameters ↑ |  |
| "strText": [“textstr”,””,””,””,””] | The text of overlay you want to show | Array[5], String |
| "textAlign": 0 | Text align, 0-align left, 1-align right | int |

    - /CGI/Custom/cameraiwhitebalance

Type: GET/PUT

| JSON | Description | Value |
| --- | --- | --- |
| { |  |  |
| "WBMode": 1, | White balance mode. 0:manual 1:auto | Int 0,1 |
| "WBr": 13, | red offset | int 0 - 100 |
| "WBg":0, | green offset | int 0 - 100 |
| "WBb": 13 | blue offset | int 0 - 100 |
| } |  |  |

    - /CGI/Preview/roimap/channels/0

| JSON | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Enable": true, |  |  |
| "AreaMax": 8, |  |  |
| "AreaConfig": [ | AreaConfig[0] |  |
| { "TopLeftX": 7, "TopLeftY", 14, "BotRightX": 32, "BotRightY": 63 }, |  |  |
| ... | AreaConfig[4] |  |
| ... | AreaConfig[7] |  |
| ] |  |  |
| } |  |  |

  - Video Manager

Web page: Recold -> Record config;

Note: Manage the video section

    - /CGI/Record/record/channels/<ch.no>

Url: http://192.168.1.41/CGI/Record/record/channels/0 (For example, address depends on yourself)

Type: GET/PUT

| JSON | Description | Value |
| --- | --- | --- |
| { |  |  |
| "RecordConfig" :{ |  |  |
| "PacketLength": 60, | Packet length | Int 1-120 |
| "PreRecord": 5, | How many seconds in advance to start recording | int (s) |
| "RecordMode": "ConfigRecord", | Mode type: "ConfigRecord", "ManualRecord", "ClosedRecord" | String |
| "Redundancy": false, | Enable redundant video | bool |
| "RecordLatch": 10, | 10 to 300 seconds |  |
| "Mask": [ | Used to tell what the video type is selected for each period of time ↓ | Array |
| [ "0x00000007", "0x00000000", "0x00000000", "0x00000000", "0x00000000", "0x00000000" ], | Mask[0] If(Mask[0][0]&(1<<0)){ // RECORD_TYPE_COMMON is on } If(Mask[0][0]&(1<<1)){ // RECORD_TYPE_MOTION is on } If(Mask[0][0]&(1<<2)){ // RECORD_TYPE_ALARM is on } | Array[6] String |
| [. . . . . . ], | Mask[1] | Array[6], String |
| [. . . . . . ], | Mask[2] | Array[6], String |
| [. . . . . . ], | Mask[3] | Array[6], String |
| [. . . . . . ], | Mask[4] | Array[6], String |
| [. . . . . . ], | Mask[5] | Array[6], String |
| [. . . . . . ] | Mask[6] | Array[6], String |
| ], | Mask ↑ |  |
| "TimeSection": [ | TimeSection ↓ |  |
| [ "1 00:00:00-24:00:00", "0 00:00:00-24:00:00", "0 00:00:00-24:00:00", "0 00:00:00-24:00:00", "0 00:00:00-24:00:00", "0 00:00:00-24:00:00" ], | TimeSection[0] For details, please see Normal JSON structure -- TimeSection | Array[6] String |
| [. . . . . . ], | TimeSection[1] | Array[6], String |
| . . . . . . |  |  |
| [. . . . . . ] | TimeSection[6] | Array[6], String |
| ] | TimeSection ↑ |  |
| }， |  |  |
| "Stream" : 0} |  |  |

    - /CGI/Record/snapshot/channels/<ch.no>

Url: http://192.168.1.41/CGI/Record/snapshot/channels/0 (For example, address depends on yourself)

Type: GET/PUT

| JSON | Description | Value |
| --- | --- | --- |
| PreSnap : 2 | Packet length | Int 1-3 |
| SnapMode : "ClosedSnap" | Mode type: "ConfigRecord", "ManualRecord", "ClosedRecord" | String |
| "Redundancy": false, | Enable redundant video | bool |
| "Mask": [ | Used to tell what the video type is selected for each period of time ↓ | Array |
| [ "0x00000007", "0x00000000", "0x00000000", "0x00000000", "0x00000000", "0x00000000" ], | Mask[0] If(Mask[0][0]&(1<<0)){ // RECORD_TYPE_COMMON is on } If(Mask[0][0]&(1<<1)){ // RECORD_TYPE_MOTION is on } If(Mask[0][0]&(1<<2)){ // RECORD_TYPE_ALARM is on } | Array[6] String |
| [. . . . . . ], | Mask[1] | Array[6], String |
| [. . . . . . ], | Mask[2] | Array[6], String |
| [. . . . . . ], | Mask[3] | Array[6], String |
| [. . . . . . ], | Mask[4] | Array[6], String |
| [. . . . . . ], | Mask[5] | Array[6], String |
| [. . . . . . ] | Mask[6] | Array[6], String |
| ], | Mask ↑ |  |
| "PeriodInterval": 1, | Unit: Seconds |  |
| "TimeSection": [ | TimeSection ↓ |  |
| [ "1 00:00:00-24:00:00", "0 00:00:00-24:00:00", "0 00:00:00-24:00:00", "0 00:00:00-24:00:00", "0 00:00:00-24:00:00", "0 00:00:00-24:00:00" ], | TimeSection[0] For details, please see Normal JSON structure -- TimeSection | Array[6] String |
| [. . . . . . ], | TimeSection[1] | Array[6], String |
| . . . . . . |  |  |
| [. . . . . . ] | TimeSection[6] | Array[6], String |
| ], | TimeSection ↑ |  |
| "Enable": false, |  |  |
| "SnapQuality": 3, | Image quality, 1:Poor,2:Average,3: Ordinary, 4:Good, 5:Excellent, 6:Best |  |
| "SnapCount": 1, |  |  |
| "SnapResolution": "720P", | Picture Resolution, See the table Table. CompressionType |  |
| "ResolutionMask":0x00000809, | resolution mask is “0x00000809”=1000 0000 1001， support D1 CIF and 720P,See the table Table. CompressionType |  |
| "SnapMaxSize": 300, | Picture Max Size |  |
| "SnapTimerMode": 0, | 0: TimingSection 1: TimeSection |  |
| "TimingInterval": 1, | Unit: Seconds |  |
| "TimingSection": [ |  |  |
| "00:00:00", | Scheduled photo-taking time, No duplicate times are allowed. |  |
| "00:00:00", | … |  |
| "00:00:00", | … |  |
| "00:00:00", | … |  |
| "00:00:00", | … |  |
| "00:00:00", | … |  |
| "00:00:00", | Scheduled photo-taking time, No duplicate times are allowed. |  |
| "00:00:00", | … |  |
| "00:00:00", | … |  |
| "00:00:00", | … |  |
| "00:00:00", | … |  |
| "00:00:00" | Scheduled photo-taking time, No duplicate times are allowed. |  |
| ], |  |  |
| "TimingSupport": 12 | Maximum supported number of timing section |  |

    - /CGI/Record/storage+<info/opt>

Get storage info from device. Only for get.  /CGI/Record/storageinfo

Url: http://192.168.1.41/CGI/Record/storageinfo (For example, address depends on yourself)

Type: GET

| JSON | Description | Value |
| --- | --- | --- |
| [ | An array |  |
| { | Info[0] |  |
| "Current": true, | Device current disk | bool |
| "Storage": "1-1", | Disk no | String |
| "StorageRemain": "0.00 MB", | Storage remain | String |
| "StorageSize": "6.42 GB", | Storage size | String |
| "StorageState": "StateErr", | Storage state | String |
| "StorageTpye": "ReadWrite", | Storage type, “ReadWrite”, “Read”, “Write”, “Redundant”, “SnapShot” | String |
| "TimeSection":"startTime: 19- 3-15 13:43: 1 - endTime: 19- 3-15 17:12:22" | Time section, “startTime: YY- M- DD HH: m: s – endTime: YY- M- DD HH: m: s” | String |
| } |  |  |
| {… …} | Info[n] |  |
| ] |  |  |

Put storage info to device. Only for put.  /CGI/Record/storageopt

Url: http://192.168.1.41/CGI/Record/storageopt (For example, address depends on yourself)

Type: PUT

| JSON | Description | Value |
| --- | --- | --- |
| { |  |  |
| “Action”:”SetType”, | Decide what to do, include: “SetType”, “Recover”, “Partition” and “Clear” |  |
| “Type”:””, | If Action=”SetType”, Type decide what type to set, include: “ReadWrite”, “ReadOnly”, “Events”, “Redundant” and “Snapshot”; If Action=”Clear”, Type=”Data”; else Type=””. |  |
| “PartitionSize”:[ | Record + Snapshot = StorageSize(Which you got in storageinfo) |  |
| { |  |  |
| “Record”:1024, | The size for storage the record videos | Int(GB) |
| “Snapshot”:1024 | The size for storage the snapshot pictures. | Int(GB) |
| } |  |  |
| ] |  |  |
| “SeriaNo”:1 | At the ‘-’’s left number in Storage(Which you got in storageinfo). | Int |
| “PartNo”:1 | At the ‘-’’s right number in Storage(Which you got in storageinfo). | Int |
| } |  |  |

- 2.20.4  /CGI/Record/schedulelist

Get the device's video calendar, that is, which days have video files..

/CGI/Record/schedulelist

Url: http://192.168.1.41/CGI/Record/storageinfo (For example, address depends on yourself)

Request:

| JSON | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Channel": 0, |  |  |
| "BeginTime":"2025-10-20 00:00:00", | The start time to be queried |  |
| "EndTime": "2025-10-20 23:59:59" | The end time to be queried |  |
| } |  |  |

Response:

[

{

"year": "2025",

"month": "10",

"days": "0x0008C000"

}

]

| JSON | Description | Value |
| --- | --- | --- |
| [ | An array |  |
| { |  |  |
| "year": 2025, | Years with video files |  |
| "month":"10", | Month with video files |  |
| "days": "0x80000" | The number of days in a month that have video files. Representing each day with a single binary digit. day = bit(n) + 1 | 0x0008C000: bit14: 15 bit15: 16 bit19: 20 |
| } |  |  |
| .... |  |  |
| ... |  |  |
| { |  |  |
| "year": 2025, | Years with video files |  |
| "month":"9", | Month with video files |  |
| "days": "0x80000" | The number of days in a month that have video files. |  |
| } |  |  |

  - PTZ fuction

    - /CGI/Ability/abilitycamera

Url: http://192.168.1.41/CGI/Ability/abilitycamera (For example, address depends on yourself)

Type: GET(only)

Description: Get the ability of camera.

| JSON | Description | Value |
| --- | --- | --- |
| { |  |  |
| “Camera”:{ |  |  |
| "AFSupport": 1, | Whether the camera supports the AF function. | 1: support; 0:not support |
| "AISupport": 0, | Whether the camera supports the AI function. | 1: support; 0:not support |
| "PTZSupport": 0, | Whether the camera supports the PTZ function. | 1: support; 0:not support |
| "WDRSupport": 1 | Whether the camera supports the WDR function. | 1: support; 0:not support |
| "TFSupport": 1, | Whether the camera supports the TF function. | 1: support; 0:not support |
| "ExSpeakerSupport":1 | Whether the camera supports external speaker | 1: support; 0:not support |
| "ExMicSupport":1 | Whether the camera supports external mic | 1: support; 0:not support |
| "FocusSupport":true, | Whether the camera supports focus control | 1: support; 0:not support |
| "ApertureSupport":true, | Whether the camera supports aperture control | 1: support; 0:not support |
| "SpeedSupport":true, | Whether the camera supports speed control | 1: support; 0:not support |
| "PresetSupport":true, | Whether the camera supports preset point settings | 1: support; 0:not support |
| "ZoomSupport":true, | Whether the camera supports zoom control | 1: support; 0:not support |
| "CruiseSupport":true | Whether the camera supports cruise | 1: support; 0:not support |
| "Reserve": "", | No ues. |  |
| } |  |  |
| } |  |  |

    - /CGI/PTZ/AFAbility

Url: http://192.168.1.41/CGI/PTZ/AFAbility (For example, address depends on yourself)

Type: GET(only)

Description: If the camera support the AF, you can get the max and min of the AF value.

| JSON | Description | Value |
| --- | --- | --- |
| { |  |  |
| "FocusMax": 2651, | The max of the focus. | Int |
| "FocusMin": 0, | The min of the focus. | Int |
| "ZoomMax": 1275, | The max of the zoom. | Int |
| "ZoomMin": 0, | The min of the zoom. | Int |
| "PresetMax": 0, |  |  |
| "PresetMin": 0, |  |  |
| "SpeedMax": 0, |  |  |
| "SpeedMin": 0, |  |  |
| "IrisLargeMax": 0, |  |  |
| "IrisLargeMin": 0, |  |  |
| } |  |  |

    - /CGI/PTZ/AFControl

Url: http://192.168.1.41/CGI/PTZ/AFControl (For example, address depends on yourself)

Description: Control all the cmd of AF.

Type: GET. Be careful, when you “GetState”, the return VarParam=1, then you should set  the button to disabled and do not send any cmd again until the VarParam=0;

| JSON | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Action": 1, | When you do “GET”, it was no user now. | 1 / 0 |
| "Command": "GetPos", | The command decides what you want when you do “GET”. - Cmd=”GetPos”, get the position(value) of zoom and focus. - Cmd=”GetSpeed”, get the step of zoom and focus. - Cmd=”GetState”, after you putying any cmd, you need check the state of device | “GetPos”, “GetSpeed”, “GetState” |
| "VarFocus": 150, | If cmd=”GetPos”, it was the position(value) of current focus; If cmd=”GetSpeed”, it was the step of current focus; If cmd=”GetState”, it was no use. | Int |
| "VarParam": 1, | If cmd=”GetPos”, it was no use; If cmd=”GetSpeed”, it was no use; If cmd=”GetState”, it was current state of device, 1=busy, 0-free. | 0/1 |
| "VarZoom": 0 | If cmd=”GetPos”, it was the position(value) of current zoom; If cmd=”GetSpeed”, it was the step of current zoom; If cmd=”GetState”, it was no use. | Int |
| } |  |  |

Type: PUT. Be careful, after you put any cmd, the device needs to take some the deal with your cmd, so you should make sure the “state” (by “GetState”) is 0(free), then you can PUT next cmd.

| JSON | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Action": 1, | If cmd=“ZoomWide”, “ZoomTile”, “Focus Near”, “FocusFar”; When you push down the button you set “Action”:1, and then you release up the button you set “Action”:0. Else set “Action”:1 | 1 / 0 |
| "Command": "SetPos", | The command decides what you want when you do “GET”. - Cmd=”SetPos”, set the position(value) of zoom and focus. - Cmd=”SetSpeed”, set the step of zoom and focus. - Cmd=”ZoomWide”, make the value of zoom smaller, the number to be added is the current zoom ‘step’. - Cmd=” ZoomTile”, make the value of zoom bigger, like “ZoomWide”. - Cmd=” FocusNear”, make the value of focus smaller, the number to be added is the current focus ‘step’. - Cmd=”FocusFar”, make the value of focus bigger, like “FocusNear”. - Cmd=” AuxFocus”, make the device to do Auto-Focus(This may take some time). - Cmd=” InitLens”, make the device to do init all the params(This may take some time). - Cmd=”Refresh”, make the device to do refresh. | “SetPos”, “SetSpeed”, “ZoomWide”, “ZoomTile”, “FocusNear”, “FocusFar”, “AuxFocus”, “InitLens”, ”Refresh” |
| "VarFocus": 150, | If cmd=”SetPos”, it was the position(value) of current focus you setted; If cmd=”SetSpeed”, it was the step of current focus you selected; Else no use. | Int |
| "VarParam": 1, | No use in PUT cmd. |  |
| "VarZoom": 0 | If cmd=”SetPos”, it was the position(value) of current zoom you setted; If cmd=”SetSpeed”, it was the step of current zoom you selected; Else no use. | Int |
| } |  |  |

    - /CGI/PTZ/PTZControl

Url: http://192.168.1.41/CGI/ PTZ/PTZControl (For example, address depends on yourself)

Description: Control all the cmd of PTZ.

Type: PUT.

| JSON | Description | Value |
| --- | --- | --- |
| { |  |  |
| "Command": "DirectionUp", | The command decides what you want when you do “GET”. - Cmd=”DirectionUp”, The surveillance camera rotates upward. - Cmd=” DirectionDown”, The surveillance camera rotates downward. - Cmd=” DirectionLeft”, The surveillance camera turns to the left. - Cmd=” DirectionRight”, The monitor camera turns to the right. - Cmd=” DirectionLeftUp”, The surveillance camera turns to the upper left. - Cmd=” DirectionLeftDown”, Monitor Camera Turns Left Downward. - Cmd=” DirectionRightUp”, The surveillance camera turns to the upper right. - Cmd=” DirectionRightDown”, Monitor cameras turn to the lower right. - Cmd=” ZoomWide”, The multiplier of the surveillance camera increases. - Cmd=” ZoomTile”, The multiple of the surveillance camera decreases. - Cmd=” FocusFar”, Far Focus of Surveillance Camera. - Cmd=” FocusNear”, Focus zooming of surveillance camera. - Cmd=” IrisLarge”, Increased aperture of surveillance camera. - Cmd=” IrisSmall”, Aperture reduction of surveillance camera. - Cmd=” SetPreset”, Setting up monitoring preset points. - Cmd=” ClearPreset”, Clear the preset points for monitoring. - Cmd=” GotoPreset”, Turn to the preset point of monitoring. - Cmd=” AddTour”, Increase monitoring cruise points. - Cmd=” DeleteTour”, Delete Monitoring Cruise Points. - Cmd=” StartTour”, Start monitoring Cruises. - Cmd=” StopTour”, End the surveillance Cruise. - Cmd=” ClearTour”, Delete Monitoring Cruise. | “DirectionUp” “DirectionDown” “DirectionLeft” “DirectionRight” “DirectionLeftUp” “DirectionLeftDown” “DirectionRightUp” “DirectionRightDown” “ZoomWide” “ZoomTile” “FocusFar” “FocusNear” “IrisLarge” “IrisSmall” “SetPreset” “ClearPreset” “GotoPreset” “AddTour” “DeleteTour” “StartTour” “StopTour” “ClearTour” |
| "Parameter": { |  |  |
| "AUX": { | Some special function, not use now. |  |
| "Number": 0, |  | int |
| "Status": "On" |  | Int |
| } |  |  |
| "Channel": 0, | When the value of the channel number is what, the operation of the channel is performed. | int |
| "MenuOpts": "Enter", |  |  |
| "Pattern": "Start", |  |  |
| "Preset": -1, | When the preset point is not set, cleared and transferred to the preset point, the value of Preset can only be zero or minus one, zero is running, minus one is stopping. Conversely, Preset's value is the value you want to enter or select. | int |
| "Step": 5, | Choose the step you want. | int |
| "Tour": 0 | Choose the cruise value between the points you want. This is used in command for tour | int |
| } |  |  |
| } |  |  |

- Normal JSON structure

  - TimeSection

| Json | Description | Value |
| --- | --- | --- |
| "TimeSection": [ | Time schedule that when the will detect the alarm. 7days and every day has 6 period |  |
| [ | Sunday |  |
| "1 00:00:00-24:00:00", | Period 1 | 1: mean enable,and 0 means disable 00:00:00-24:00:00: means time period |
| "0 00:00:00-24:00:00", | Period 2 |  |
| "0 00:00:00-24:00:00", | Period 3 |  |
| "0 00:00:00-24:00:00", | Period 4 |  |
| "0 00:00:00-24:00:00", | Period 5 |  |
| "0 00:00:00-24:00:00" | Period 6 |  |
| ], |  |  |
| [ ...... | Monday |  |
| ], |  |  |
| [ | Tuesday |  |
| ...... |  |  |
| ], |  |  |
| [ |  |  |
| ...... |  |  |
| ], |  |  |
| [ |  |  |
| ...... |  |  |
| ], |  |  |
| [ |  |  |
| ...... |  |  |
| ], |  |  |
| [ |  |  |
| ...... |  |  |
| ], |  |  |
| ], |  |  |

  - Server

| Json | Description |  |  |  |  | Value |
| --- | --- | --- | --- | --- | --- | --- |
| "Server": { |  |  |  |  |  |  |
| "Address": "0x0100060A", |  | The server address |  |  |  |  |
| "Anonymity": false, |  |  |  |  | false true |  |
| "Name": "hphwebservice.oray.net", |  | The real domain name of DDNS |  |  |  |  |
| "Password": "", |  | The password or server account |  |  | Password is use DES + BASE64 encode. |  |
| "Port": 80, |  | The server port |  |  |  |  |
| "UserName": "" |  | The user name of server account |  |  |  |  |
| } |  |  |  |  |  |  |

  - EventHandler

| Json | Description | Value |
| --- | --- | --- |
| "EventHandler": { | What will do when alarm happen. |  |
| "AlarmInfo": "", | No use |  |
| "AlarmOutEnable": false, | For Alarm out link.If the device no alarm out port than no care. | true: open alarm out false:close alarm out |
| "AlarmOutLatch": 10, | The delay time |  |
| "AlarmOutMask": "0x00000000", | Mask for Alarm out channel, use bit to express. | 0x0001: bit0=1 means channel 1 is on; 0x0003: bit0,bit1=1. means channel 0 and channel 1 is on. |
| "BeepEnable": false, | Beep Open or close |  |
| "EventLatch": 1, | Delay |  |
| "FTPEnable": false, | Upload by FTP or not | True or false |
| "LogEnable": false, | No use |  |
| "MailEnable": false, | Send Email or not |  |
| "MatrixEnable": false, | No use |  |
| "MatrixMask": "0x00000000", | No use |  |
| "MessageEnable": false, | For DVR, show the message box or not |  |
| "MsgtoNetEnable": false, | No use |  |
| "MultimediaMsgEnable": false, | No use |  |
| "PtzEnable": false, | PTZ Preset,Need device support |  |
| "PtzLink": [ |  |  |
| [ |  |  |
| "None",0 |  |  |
| ], |  |  |
| ], |  |  |
| "RecordEnable": false, | Alarm record or not |  |
| "RecordLatch": 10, | Delay time |  |
| "RecordMask": "0x00000000", | Mask for channel, which channel to record |  |
| "ShortMsgEnable": false, | No use |  |
| "ShowInfo": false, | No use |  |
| "ShowInfoMask": "0x00000000", | No use |  |
| "SnapEnable": false, | Snap or not ,need the device support snap |  |
| "SnapShotMask": "0x00000000", | Which channel to snap, also use bit to express. |  |
| "TimeSection": [ |  |  |
| ] |  |  |
| "TipEnable": false, | DVR: Message box IPC: NO USE |  |
| "TourEnable": false, | NO use for IPC |  |
| "TourMask": "0x00000000", |  |  |
| "VoiceEnable": false, | No use |  |
| "bLightEn": false | Light open prompt, Light off prompt |  |
| } |  |  |

  - AuthorityList

The Authority of the group or user has to do for the device.

Json

"AuthorityList" {

"ShutDown",; shut down the device. "ChannelTitle",; Change channel title Web page:Camera->OutputModel. "RecordConfig",; Like change the record time and which mode to save record, handle,configure or stop. For the device has SD card.. "Backup",; Backup record from device. "StorageManager",; The Authority to Format HDD and HDD Partition.Need SD card or HDD. "Account",; User manager, add, delete user or group. Web page: System Config->System Tool->User. "SysInfo",; Authority to get the system information. Web page : System Info->System Info. "QueryLog",; Authority to query log. Web page: System Info->Log Info. "DelLog",; Delete the log Web page: System Info->Log Info. "SysUpgrade",; Upgrade the system. "AutoMaintain",; Automatic Maintenance Web page: System Config->System Tool->Auto Maintain. "GeneralConfig",; Including 3 page: 1:System Config->System Config 2:System Config->System Config->General Config 3:System Config->System Config->NTP Setting. "EncodeConfig",; Encode Setting Web page： Camera->Encode Config. "NetConfig",; Include netcommon configure and net service configure. "AlarmConfig",; All the alarm setting, including motion, alarm in alarm out and abnormal alarm. "VideoConfig",. "DefaultConfig",; Restore the device. "Talk_01",; Authority to talk, need the device support audio. "DigitalManager",; Design for NVR ,not set yet. "Monitor_01",; Real time monitor for channel1. "Replay_01"; Playback function of channel 1. ],.

- Error Code

Any Get or Set function the device will reply the HTTP info, and the info will give out the operation success or fail, also the reason why fail.

{ "RequestURL" : "/CGI/UserManager/users/bbbb", "statusCode" : 1, "statusString" : "OK" }

| Field | Description |
| --- | --- |
| statusCode | status code |
| Statusstring | Specific error |

  - StatusCode

| Specific value | Description |
| --- | --- |
| STATUS_OK =1 | Operate success |
| STATUS_DEVICE_ERROR=2 | Device error |
| STATUS_INVALID_OPT=3 | Invalid operation |
| STATUS_CONFIG_FAILED=4 | Get or set configuration,just see the Status string to get the real error. |

  - Statusstring

| Specific value | Description |
| --- | --- |
| OK | Operate success |
| OK And Reboot | Operate success and need to reboot device |
| Device Error | Device error |
| Invalid Operation | Invalid Operation |
| GetConfig Error | Get configuration error |
| Channel Error | Channel error |
| User be userd | The user name is used |
| Password Not Match | The password error |
| UserName Error | The username error |
| OPT Failed | Operate failed |
| SetConfig Error | set configuration error |
| Config Error | Configuration error |

//Provide identification codes for corresponding WEB translations to facilitate WEB translation into various languages; any modifications need to be synchronized with the WEB.

| Specific value | Description |
| --- | --- |
| "100" | "Reserved User" |
| "101" | "Reserved User'authority can't be changed" |
| "102" | "Reserved group can not be deleted" |
| "103" | "The authority large than The Group" |
| "104" | "The Group authority less than Group's users" |
| "105" | "Group has users, cannot delete" |
| "106" | "Password Not Match" |
| "107" | "Restore failed" |
| "108" | "The list is full, up to 64 IP addresses" |
| "109" | "failed" |
| "110" | "NoAuthority" |
| "111" | "Cannot add the same user" |
| "112" | "Cannot modify the same user" |
| "113" | "Cannot add the same group" |
| "114" | "Cannot modify the same group" |

- Function through OCX

| Function | Web Page | Description |
| --- | --- | --- |
| LIVEVIDEO | Preview |  |
| Record on Live view | Preview |  |
| Snap on Live view | Preview |  |
| Local Config | Local configure | Change the file path of record or picture. |
| Region selection for Motion | Alarm->Alarm config->Motion detection |  |
| Output Model | Camera->OutputModel->outputmodel | For change channel name, and title place |
| DisplayConfig | Camera->OutputModel->DisplayConfig | For ISP, Picture color |
| Privacy Masking | Camera->OutputModel->Privacy Masking |  |
| Text overlay | Camera->OutputModel->Overlay |  |
| Import/Export | System Config->System tool->System Maintain |  |
| Upgrade | System Config->System tool->System Maintain |  |
| Color Setting | Preview->Image Adjustment |  |
| Playback function | Playback(For the device has SD card) |  |

- Web Socket Server

  - General Cmd

typedef enum {

WEBSOCKET_CMD_NONE = 0,

//Get camera encoding properties

WEBSOCKET_CMD_GET_CAMERA_ENCODE = 1, //Get encoding properties

WEBSOCKET_CMD_GET_CAMERA_PRIVACY_MASK = 2, //Obtain privacy masking

WEBSOCKET_CMD_SET_CAMERA_PRIVACY_MASK = 3, //Set privacy mask

WEBSOCKET_CMD_GET_CAMERA_MOTION_DETECT = 4,//Enable motion detection

WEBSOCKET_CMD_SET_CAMERA_MOTION_DETECT = 5,//Modify motion detection

WEBSOCKET_CMD_GET_CAMERA_OVERLAY = 22,//Get video overlay

WEBSOCKET_CMD_MODIFY_CAMERA_OVERLAY =23,//Modify video overlay settings

WEBSOCKET_CMD_SET_CAMERA_PTZ_CTRL = 24, //Gimbal Control

//Configure to Export

WEBSOCKET_CMD_EXPORT_SYS_CONFIG_PACK = 259,//package export configuration

//Import configuration

WEBSOCKET_CMD_IMPORT_SYS_CONFIG = 261,//Import configuration

WEBSOCKET_CMD_IMPORT_SYS_CONFIG_PROGRESS = 262,//Import configuration progress

//Manual upgrade

WEBSOCKET_CMD_REMOTE_UPGRADE = 270,//remote upgrade

WEBSOCKET_CMD_REMOTE_UPGRADE_PROGRESS = 271,//Remote upgrade reception progress

//preview

WEBSOCKET_CMD_REQUEST_VIDEO_PLAY = 800, //Request to preview the video

//Peplay

WEBSOCKET_CMD_REQUEST_RECORD_PLAY = 900, //Request to play the video

WEBSOCKET_CMD_SYNC_RECORD_TIMETICK = 901, //Synchronized video timestamp

WEBSOCKET_CMD_RECORD_PLAY_CONTROL = 902, //Replay Control

WEBSOCKET_CMD_RECORD_PLAY_END = 903, //End of file playback

//Video download

WEBSOCKET_CMD_RECORD_DOWLOAD = 910, //Video download

WEBSOCKET_CMD_RECORD_DOWLOAD_END = 911, //Video download complete

WEBSOCKET_CMD_REQ_CLOSE_SOCKET = 999, //the server actively closes

}WEBSOCKET_CMDID;

  - General Resp

typedef enum{

WEB_RET_OK              = 0,  /*success*/

WEB_RET_FAILURE         = 1,  /*Fail*/

WEB_RET_ERR_PWD         = 2,  /*Incorrect password*/

WEB_RET_CMD_UNKNOW      = 3,  /*Unknown command type*/

WEB_RET_CMD_JSON_ERROR      = 4,  /*Json Error*/

WEB_RET_CMD_NO_LOGIN      = 5,  /*Client not logged in*/

WEB_RET_CMD_ERR_DEVMAX      = 6,  /*Adding devices exceeds the number of channels*/

WEB_RET_CMD_NOT_SUPPORT 	= 7, /*This device does not support this setting.*/

WEB_RET_CMD_OK_REBOOT 	= 8, /*Setup successful, the device will restart to take effect.*/

WEB_RET_CMD_USERNAME_ERROR 	= 9, /*incorrect username*/

WEB_RET_CMD_UPLOADING 	= 10, /*uploading file*/

WEB_RET_CMD_CONNECTED_TIMEOUT 	= 11, /*connection timed out*/

WEB_RET_CMD_UPGRADE_FAILURE 	= 12, /*upgrade failed*/

WEB_RET_CMD_UPGRADE_INVALID 	= 13, /*upgrade package verification failed*/

WEB_RET_CMD_CHANNEL_ISPLAYING 	= 14, /* the channel is in playback mode*/

WEB_RET_CMD_LOGOUT 	= 15, /*web need to log out*/

}WEBSOCKET_RET_CODE;

  - Video Play

Req: { "cmd" : REQUEST_VIDEO_PLAY, "data" : { "chn": 0, "streamType": 0}}

  - Record Play

Req:

{

"cmd" : REQUEST_RECORD_PLAY,

"data" : {

" type": 0, " recordType": 0,

" year": 0, " month": 0, " day": 0,

" hour": 0, " minute": 0," second ": 0

}}

  - Record Play Ctrl

//Control types 1: Play 2: Pause 3: Stop 4: Slow motion 5: Fast forward

Req: { "cmd" : RECORD_PLAY_CONTROL, "data" : { "type": 0}}

  - Record Dowload

Req:

{

"cmd" : RECORD_DOWLOAD,

"data" : {

"year": 0, " month": 0, " day": 0,

"playTs":{"hour ": 0,"minute": 0,"second": 0},

"playTs":{"hour ": 0,"minute": 0,"second": 0},

}}

  - Remote Upgrade

Req: {"cmd": REMOTE_UPGRADE,"data":{"streamType":0}}

  - Camera Encode

Req: {"cmd":GET_CAMERA_ENCODE,"data":{"filename ":0, "size":0, "md5":0}}

  - Get Camera Overlay

{ "cmd" : GET_CAMERA_OVERLAY, "data" : { "timeRect" : { "TimeEnable" : 1, "bottom" : 19, "left" : 0, "right" : 18, "top" : 0 }, "titleRect" : { "TitleEnable" : 1, "chnTitle" : "0123456", "bottom" : 229, "left" : 0, "right" : 3, "top" : 229 }, "customRect" : { "CustomEnable" : 1, "customTitle" : "ABCDEF", "bottom" : 20, "left" : 260, "right" : 450, "top" : 0 } }

  - Set Camera Overlay

{"cmd":MODIFY_CAMERA_OVERLAY,"data":{"chnTimeEnable":1,"timeRect":{"bottom":20,"left":260,"right":450,"top":0},"chnTitle":"0123456","chnTitleEnable":1,"titleRect":{"bottom":233,"left":3,"right":0,"top":213},"customTitle":"ABCDEF","CustomEnable":1,"customRect":{"bottom":20,"left":260,"right":450,"top":0}}}

  - Get Camera Privacy Mask

未使用

  - Set Camera Privacy Mask

未使用

  - Get Camera Motion Detect

{"cmd": GET_CAMERA_MOTION_DETECT,"data":{"lines": 0,"rows": 0,"region":[{"col":0,"row":0,"value":1},{"col":1,"row":0,"value":1},{"col":2,"row":0,"value":1},{"col":3,"row":0,"value":1},{"col":4,"row":0,"value":1, …}]}}

  - Set Camera Motion Detect

{"cmd":SET_CAMERA_MOTION_DETECT,"data":{"region":[{"col":0,"row":0,"value":1},{"col":1,"row":0,"value":1},{"col":2,"row":0,"value":1},{"col":3,"row":0,"value":1},{"col":4,"row":0,"value":1, …}]}}

  - Set Camera PTZ Ctrl

control : RIGHT = 1, LEFT = 2, UP = 3, DOWN = 4, ALL= 5

Req: { "cmd" : SET_CAMERA_PTZ_CTRL, "data" : { " control": 1}}

- Customize function

List here some non-generic functions’ description.

- Device’s password init

When the device is initialized to the factory state, before you login on web, you need to modify password at first.